ARCADY 7

Version: 7.0.0.99 [10 July 2009] © Copyright Transport Research Laboratory 2009

For sales and distribution information, program advice and maintenance, contact TRL: Tel: +44 (0)1344 770758 E-mail: software@trl.co.uk Web: www.trlsoftware.co.uk

The users of this computer program for the solution of an engineering problem are in no way relieved of their responsibility for the correctness of the solution

File: (new file)
Report generation date: 29/03/2018 13:42:02

File summary

File Description Title; (untitled). Location. Site Number. Date; 29/03/2018. Version. Status; (new file). Identifier. Client. Jobnumber. Enumerator; ADF\yxl879. Description. Results Upto Date; True.

Analysis Options

| RFC Threshold | Vehicle Length (m) | Do Queue Variations |
| --- | --- | --- |
| 0.85 | 5.75 |  |

Sorting and Display

| Show Arm Names | Arm Grouping | Sorting Direction | Sorting Type | Data Matrix Style | Time Style |
| --- | --- | --- | --- | --- | --- |
|  | Order | Ascending | Numerical | By Destination | Absolute Time |

Units

| Distance Units | Speed Units | Traffic Units Input | Traffic Units Results | Flow Units | Average Delay Units | Total Delay Units | Rate Of Delay Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | kph | PCU | PCU | perHour | min | -Min | perMin |

A1 - (Default Analysis Set) - D1 - (Scenario1), AM

Data Errors and Warnings

No errors or warnings

Analysis Set Details

| Name | Description | Include In Report | Use Specific Demand Set | Demand Set | Locked | Network Flow Scaling Factor (%) | Network Capacity Scaling Factor (%) | Reason For Scaling Factors |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Default Analysis Set) |  | Yes |  | (D1) |  | 100.000 | 100.000 |  |

Demand Set Details

| Name | Scenario Name | Time Period Name | Description | Locked | Run Automatically | Use Relationship | Relationship | Start Time (HH:mm) | Finish Time (HH:mm) | Time Period Length (min) | Time Segment Length (min) | Traffic Profile Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Scenario1), AM | (Scenario1) | AM |  |  | Yes |  |  | 08:00 | 09:00 | 60 | 15 | FLAT |

Roundabout Network

Roundabout Type(s)

| ID | Name | Arm Order | Roundabout Type | Grade Separated | Large Roundabout | Do Geometric Delay |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | (untitled) | 1,2,3 | Standard |  |  |  |

Roundabout Network Options

| Driving Side | Lighting | Road Surface | In London |
| --- | --- | --- | --- |
| Left | Normal/unknown | ((Mini-roundabouts only)) |  |

Arms

Arms

| ID | Name | Description |
| --- | --- | --- |
| 1 | (untitled) |  |
| 2 | (untitled) |  |
| 3 | (untitled) |  |

Capacity Options

| Arm | Minimum Capacity (PCU/hr) | Maximum Capacity (PCU/hr) | Assume Flat Start Profile | Initial Queue (PCU) |
| --- | --- | --- | --- | --- |
| 1 | 0.00 | 99999.00 |  | 0.00 |
| 2 | 0.00 | 99999.00 |  | 0.00 |
| 3 | 0.00 | 99999.00 |  | 0.00 |

Standard Geometry

| Arm | V - Approach road half-width (m) | E - Entry width (m) | l' - Effective flare length (m) | R - Entry radius (m) | D - Inscribed circle diameter (m) | PHI - Conflict (entry) angle (deg) | Exit Only |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4.90 | 7.90 | 14.00 | 52.00 | 42.00 | 24.00 |  |
| 2 | 5.00 | 8.75 | 20.00 | 44.50 | 42.00 | 29.70 |  |
| 3 | 3.60 | 8.50 | 18.00 | 21.00 | 42.00 | 30.00 |  |

Pedestrian Crossings

| Arm | Crossing Type |
| --- | --- |
| 1 | Pelican |
| 2 | None |
| 3 | None |

Pelican/ Puffin Crossings

| Arm | Amber time preceding red (s) | Amber time regarded as green (s) | Time from traffic red start to green man start (s) | Time period green man shown (s) | Clearance Period (s) | Traffic minimum green (s) | Space between crossing and junction entry (PCU) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.00 | 2.90 | 1.00 | 6.00 | 6.00 | 7.00 | 0.00 |

Arm Slope/ Intercept and Capacity

Slope and Intercept used in model

| Arm | Enter Directly | Slope | Intercept (PCU/hr) | Final Slope | Final Intercept (PCU/hr) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ((calculated)) | ((calculated)) | 0.737 | 2126.981 |
| 2 |  | ((calculated)) | ((calculated)) | 0.762 | 2287.379 |
| 3 |  | ((calculated)) | ((calculated)) | 0.675 | 1888.673 |

The slope and intercept shown above include any corrections and adjustments.

Traffic Flows

Demand Set Data Options

| Default Vehicle Mix | Vehicle Mix Varies Over Time | Vehicle Mix Varies Over Turn | Vehicle Mix Varies Over Entry | Vehicle Mix Source | PCU Factor for a HV (PCU) | Default Turning Proportions | Estimate from entry/exit counts | Turning Proportions Vary Over Time | Turning Proportions Vary Over Turn | Turning Proportions Vary Over Entry |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yes | Yes | HV Percentages | 2.00 |  |  |  | Yes | Yes |

Entry Flows

General Flows Data

| Arm | Profile Type | Use Turning Counts | Average Demand Flow (PCU/hr) | Flow Scaling Factor (%) | PHF |
| --- | --- | --- | --- | --- | --- |
| 1 | FLAT | Yes | 1350.00 | 100.000 | 1.00 |
| 2 | FLAT | Yes | 1350.00 | 100.000 | 1.00 |
| 3 | FLAT | Yes | 900.00 | 100.000 | 1.00 |

Pedestrian Flows

General Flows Data

| Arm | Profile Type | Average Pedestrian Flow (Ped/hr) |
| --- | --- | --- |
| 1 | FLAT | 0.00 |
| 2 | (FLAT) | (0.00) |
| 3 | (FLAT) | (0.00) |

Direct/Resultant Flows

Direct Flows Data

| Time Segment | Arm | Direct Demand Entry Flow (PCU/hr) | DirectDemandEntryFlowInPCU (PCU/hr) | Direct Demand Exit Flow (PCU/hr) | Direct Demand Pedestrian Flow (Ped/hr) |
| --- | --- | --- | --- | --- | --- |
| 08:00-08:15 | 1 | 1350.00 | 1350.00 | N/A | 0.00 |
| 08:00-08:15 | 2 | 1350.00 | 1350.00 | N/A | N/A |
| 08:00-08:15 | 3 | 900.00 | 900.00 | N/A | N/A |
| 08:15-08:30 | 1 | 1350.00 | 1350.00 | N/A | 0.00 |
| 08:15-08:30 | 2 | 1350.00 | 1350.00 | N/A | N/A |
| 08:15-08:30 | 3 | 900.00 | 900.00 | N/A | N/A |
| 08:30-08:45 | 1 | 1350.00 | 1350.00 | N/A | 0.00 |
| 08:30-08:45 | 2 | 1350.00 | 1350.00 | N/A | N/A |
| 08:30-08:45 | 3 | 900.00 | 900.00 | N/A | N/A |
| 08:45-09:00 | 1 | 1350.00 | 1350.00 | N/A | 0.00 |
| 08:45-09:00 | 2 | 1350.00 | 1350.00 | N/A | N/A |
| 08:45-09:00 | 3 | 900.00 | 900.00 | N/A | N/A |

Turning Proportions

Turning Counts or Proportions (PCU/hr) - Roundabout 1 (for whole period)

|  | To |  |  |  |
| --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 |
|  | 1 | 0.00 | 900.00 | 450.00 |
|  | 2 | 900.00 | 0.00 | 450.00 |
|  | 3 | 450.00 | 450.00 | 0.00 |

Turning Proportions (PCU) - Roundabout 1 (for whole period)

|  | To |  |  |  |
| --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 |
|  | 1 | 0.00 | 0.67 | 0.33 |
|  | 2 | 0.67 | 0.00 | 0.33 |
|  | 3 | 0.50 | 0.50 | 0.00 |

Vehicle Mix

Average PCU Per Vehicle - Roundabout 1 (for whole period)

|  | To |  |  |  |
| --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 |
|  | 1 | 1.00 | 1.05 | 1.05 |
|  | 2 | 1.05 | 1.00 | 1.05 |
|  | 3 | 1.05 | 1.05 | 1.00 |

Heavy Vehicle Percentages - Roundabout 1 (for whole period)

|  | To |  |  |  |
| --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 |
|  | 1 | 0.00 | 5.00 | 5.00 |
|  | 2 | 5.00 | 0.00 | 5.00 |
|  | 3 | 5.00 | 5.00 | 0.00 |

Results

Results Summary

| Arm | Max RFC | Max Delay (min) | Max Queue (PCU) | Max LOS | Total Demand (PCU/hr) | Total Arrivals (PCU) | Total Queueing Delay (PCU-min) | Average Queueing Delay (min) | Rate Of Queueing Delay (PCU-min/min) | Inclusive Queueing Total Delay (PCU-min) | Inclusive Queueing Average Delay (min) | Slope | Intercept (PCU/hr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.75 | 0.14 | 3.15 | A | 1350.00 | 1350.00 | 182.59 | 0.14 | 3.04 | 182.76 | 0.14 | 0.737 | 2126.981 |
| 2 | 0.69 | 0.11 | 2.37 | A | 1350.00 | 1350.00 | 138.76 | 0.10 | 2.31 | 138.85 | 0.10 | 0.762 | 2287.379 |
| 3 | 0.70 | 0.17 | 2.45 | A | 900.00 | 900.00 | 142.16 | 0.16 | 2.37 | 142.31 | 0.16 | 0.675 | 1888.673 |

Main Results

Main results: (08:00-08:15)

| Arm | Demand (PCU/hr) | Arrivals (PCU) | Entry Flow (PCU/hr) | Exit Flow (PCU/hr) | Circulating Flow (PCU/hr) | Pedestrian Demand (Ped/hr) | Capacity (PCU/hr) | Saturation Capacity (PCU/hr) | RFC | Start Queue (PCU) | End Queue (PCU) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1350.00 | 337.50 | 1337.82 | 1339.08 | 445.26 | 0.00 | 1798.95 | 1737.29 | 0.750 | 0.00 | 3.05 |
| 2 | 1350.00 | 337.50 | 1340.72 | 1337.14 | 445.94 | 0.00 | 1947.75 | 1846.34 | 0.693 | 0.00 | 2.32 |
| 3 | 900.00 | 225.00 | 890.53 | 892.85 | 893.82 | 0.00 | 1285.41 | 1057.91 | 0.700 | 0.00 | 2.37 |

Main results: (08:15-08:30)

| Arm | Demand (PCU/hr) | Arrivals (PCU) | Entry Flow (PCU/hr) | Exit Flow (PCU/hr) | Circulating Flow (PCU/hr) | Pedestrian Demand (Ped/hr) | Capacity (PCU/hr) | Saturation Capacity (PCU/hr) | RFC | Start Queue (PCU) | End Queue (PCU) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1350.00 | 337.50 | 1349.71 | 1349.79 | 449.88 | 0.00 | 1795.55 | 1737.29 | 0.752 | 3.05 | 3.12 |
| 2 | 1350.00 | 337.50 | 1349.86 | 1349.69 | 449.90 | 0.00 | 1944.73 | 1846.34 | 0.694 | 2.32 | 2.35 |
| 3 | 900.00 | 225.00 | 899.76 | 899.86 | 899.91 | 0.00 | 1281.30 | 1057.91 | 0.702 | 2.37 | 2.43 |

Main results: (08:30-08:45)

| Arm | Demand (PCU/hr) | Arrivals (PCU) | Entry Flow (PCU/hr) | Exit Flow (PCU/hr) | Circulating Flow (PCU/hr) | Pedestrian Demand (Ped/hr) | Capacity (PCU/hr) | Saturation Capacity (PCU/hr) | RFC | Start Queue (PCU) | End Queue (PCU) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1350.00 | 337.50 | 1349.91 | 1349.94 | 449.96 | 0.00 | 1795.48 | 1737.29 | 0.752 | 3.12 | 3.14 |
| 2 | 1350.00 | 337.50 | 1349.96 | 1349.90 | 449.97 | 0.00 | 1944.68 | 1846.34 | 0.694 | 2.35 | 2.36 |
| 3 | 900.00 | 225.00 | 899.93 | 899.96 | 899.97 | 0.00 | 1281.26 | 1057.91 | 0.702 | 2.43 | 2.45 |

Main results: (08:45-09:00)

| Arm | Demand (PCU/hr) | Arrivals (PCU) | Entry Flow (PCU/hr) | Exit Flow (PCU/hr) | Circulating Flow (PCU/hr) | Pedestrian Demand (Ped/hr) | Capacity (PCU/hr) | Saturation Capacity (PCU/hr) | RFC | Start Queue (PCU) | End Queue (PCU) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1350.00 | 337.50 | 1349.96 | 1349.97 | 449.98 | 0.00 | 1795.47 | 1737.29 | 0.752 | 3.14 | 3.15 |
| 2 | 1350.00 | 337.50 | 1349.98 | 1349.95 | 449.99 | 0.00 | 1944.67 | 1846.34 | 0.694 | 2.36 | 2.37 |
| 3 | 900.00 | 225.00 | 899.97 | 899.98 | 899.99 | 0.00 | 1281.25 | 1057.91 | 0.702 | 2.45 | 2.45 |

Queueing Delay Results

Queueing Delay results: (08:00-08:15)

| Arm | Queueing Total Delay (PCU-min) | Queueing Rate Of Delay (PCU-min/min) | Average Delay Per Arriving Vehicle (min) | Unsignalised Level Of Service | Signalised Level Of Service |
| --- | --- | --- | --- | --- | --- |
| 1 | 42.09 | 2.81 | 0.133 | A | A |
| 2 | 32.75 | 2.18 | 0.102 | A | A |
| 3 | 32.74 | 2.18 | 0.156 | A | A |

Queueing Delay results: (08:15-08:30)

| Arm | Queueing Total Delay (PCU-min) | Queueing Rate Of Delay (PCU-min/min) | Average Delay Per Arriving Vehicle (min) | Unsignalised Level Of Service | Signalised Level Of Service |
| --- | --- | --- | --- | --- | --- |
| 1 | 46.36 | 3.09 | 0.141 | A | A |
| 2 | 35.11 | 2.34 | 0.106 | A | A |
| 3 | 36.09 | 2.41 | 0.165 | A | A |

Queueing Delay results: (08:30-08:45)

| Arm | Queueing Total Delay (PCU-min) | Queueing Rate Of Delay (PCU-min/min) | Average Delay Per Arriving Vehicle (min) | Unsignalised Level Of Service | Signalised Level Of Service |
| --- | --- | --- | --- | --- | --- |
| 1 | 46.96 | 3.13 | 0.141 | A | A |
| 2 | 35.40 | 2.36 | 0.106 | A | A |
| 3 | 36.58 | 2.44 | 0.165 | A | A |

Queueing Delay results: (08:45-09:00)

| Arm | Queueing Total Delay (PCU-min) | Queueing Rate Of Delay (PCU-min/min) | Average Delay Per Arriving Vehicle (min) | Unsignalised Level Of Service | Signalised Level Of Service |
| --- | --- | --- | --- | --- | --- |
| 1 | 47.19 | 3.15 | 0.141 | A | A |
| 2 | 35.51 | 2.37 | 0.106 | A | A |
| 3 | 36.76 | 2.45 | 0.165 | A | A |

Overview: Standard Roundabout Geometry

Standard Geometry

| Arm | V - Approach road half-width (m) | E - Entry width (m) | l' - Effective flare length (m) | R - Entry radius (m) | D - Inscribed circle diameter (m) | PHI - Conflict (entry) angle (deg) | Exit Only | Final Slope | Final Intercept (PCU/hr) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4.90 | 7.90 | 14.00 | 52.00 | 42.00 | 24.00 |  | 0.737 | 2126.981 |
| 2 | 5.00 | 8.75 | 20.00 | 44.50 | 42.00 | 29.70 |  | 0.762 | 2287.379 |
| 3 | 3.60 | 8.50 | 18.00 | 21.00 | 42.00 | 30.00 |  | 0.675 | 1888.673 |

Overview: Time Segment Results

Time Segment Results

| Time Segment | Arm | Demand (PCU/hr) | Capacity (PCU/hr) | RFC | Pedestrian Demand (Ped/hr) | Start Queue (PCU) | End Queue (PCU) | Queueing Total Delay (PCU-min) | Geometric Total Delay (PCU-min) | Average Delay Per Arriving Vehicle (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08:00-08:15 | 1 | 1350.00 | 1798.95 | 0.750 | 0.00 | 0.00 | 3.05 | 42.09 | (0.00) | 0.133 |
| 08:00-08:15 | 2 | 1350.00 | 1947.75 | 0.693 | 0.00 | 0.00 | 2.32 | 32.75 | (0.00) | 0.102 |
| 08:00-08:15 | 3 | 900.00 | 1285.41 | 0.700 | 0.00 | 0.00 | 2.37 | 32.74 | (0.00) | 0.156 |
| 08:15-08:30 | 1 | 1350.00 | 1795.55 | 0.752 | 0.00 | 3.05 | 3.12 | 46.36 | (0.00) | 0.141 |
| 08:15-08:30 | 2 | 1350.00 | 1944.73 | 0.694 | 0.00 | 2.32 | 2.35 | 35.11 | (0.00) | 0.106 |
| 08:15-08:30 | 3 | 900.00 | 1281.30 | 0.702 | 0.00 | 2.37 | 2.43 | 36.09 | (0.00) | 0.165 |
| 08:30-08:45 | 1 | 1350.00 | 1795.48 | 0.752 | 0.00 | 3.12 | 3.14 | 46.96 | (0.00) | 0.141 |
| 08:30-08:45 | 2 | 1350.00 | 1944.68 | 0.694 | 0.00 | 2.35 | 2.36 | 35.40 | (0.00) | 0.106 |
| 08:30-08:45 | 3 | 900.00 | 1281.26 | 0.702 | 0.00 | 2.43 | 2.45 | 36.58 | (0.00) | 0.165 |
| 08:45-09:00 | 1 | 1350.00 | 1795.47 | 0.752 | 0.00 | 3.14 | 3.15 | 47.19 | (0.00) | 0.141 |
| 08:45-09:00 | 2 | 1350.00 | 1944.67 | 0.694 | 0.00 | 2.36 | 2.37 | 35.51 | (0.00) | 0.106 |
| 08:45-09:00 | 3 | 900.00 | 1281.25 | 0.702 | 0.00 | 2.45 | 2.45 | 36.76 | (0.00) | 0.165 |