CHAPTER – 3

PROPOSED METHODOLOGY

3.1 MATHEMATICAL MODEL OF CARBON DIOXIDE ABSORPTION PROCESS

Aspen HYSYS version 8.6 by selecting the Kent-Eisenberg vapour/liquid equilibrium model have been applies in this study.

3.1.1 Cost of CO2 removal – CCR

The key economic performance indicators used in this are the levelised cost of hydrogen (LCOH), the cost of CO2 avoided (CCA) and removal (CCR). These costs are calculated by the following equations:

LCOH =(TOC * FCF + FOM)(CF * 8760)+ FC * HR+ VOM                         (3.1)

CCA =LCOHLCOH-LCOHNon CCSCO2RemovedRemoved                                                            (3.2)

Where TOC is the total overnight capital cost, FOM is the fixed operating and maintenance costs, VOM is the variable operating and maintenance costs, FC is the fuel costs, CF is the capacity factor, HR is the net heat rate of the plant, and FCF is the fixed charge factor

This study has been divided into 2 sections –

3.2 TECHNO-ECONOMIC ANALYSIS OF CO2 CAPTURE CONCEPTS

3.2.1 Sections

This section will further divide into 4 parts which is as follows –

- Part – 1 - Heat recovery networks for industrial surplus heat for carbon dioxide absorption cost calculation

- Part – 2 - CO2 capture using excess heat at a cement plant based on Amine

- Part – 3 - Cost Estimation of CO2 Absorption Plants for CO2 Mitigation cost estimation tool and identifies the crucial technical and economic factors

- Part – 4 - for CO2 Capture at a Cement Plant based on Steam Production Options

3.2.2 Methodology

The contribution to the CO2stCap Project

The project's numerous research partners are given tasks using the CO2stCap. Workshops and consortium meetings were held in addition to industry visits.

These project meetings were where some of the project choices and process parameters were made. Together with the project's research collaborators, the approach for this study was developed. Additionally, there were plans to formalize the described Enhanced Detailed Factor cost estimating approach.

3.2.3 Techno-Economic methodology

The study given in this thesis is based on the following procedure for conducting a process' techno-economic analysis:

- Simulated the process using Aspen Hysys to derive mass and energy balances.

- Provided a simplified flow diagram of the process as well as an estimate of its scope.

- Equipment dimensioning via the use of data from simulations;

- Cost estimating through the use of specific individual installation factors for each piece of equipment;

- Process optimization through the use of sensitivity analysis on the most relevant technical or economic elements.

The following section will provide an in-depth explanation of the methodology's primary components, beginning with the scope analysis and continuing on to in-depth analyses of the capital expenditures and operating expenses for a process similar to an amine-based CO2 capture plant, among other examples.

An Enhanced Detailed Factor EDF technique is shown here for the purpose of capital cost estimating. This method takes the same approach as the Individual Factor and Sub-factor Estimating method, which is detailed in the book written by Gerrard. Readers of the open literature do not always have access to the technical details of the equipment, the source of the cost of the equipment, and the installation factors; however, all of these so-called "hidden factors" are revealed in a clear and distinct manner in the EDF method that is being presented. Installation factors, sometimes referred to as "improved detailed installation factors" in this work, are at the core of this work's EDF approach for cost estimate and are explained in depth. Nils Henrik Eldrup is responsible for the development of these "improved detailed installation variables." He did this over the course of a number of years while working on a variety of projects. The terminology for cost estimate is the same in this technique as it was in the preceding methods developed by NETL and IEAGHG; the only variation is in the calculation process that is used. The EDF method has several advantages over other methods, including greater transparency in cost estimates, a higher level of accuracy in early-stage cost estimates, an emphasis on individual process equipment for optimization, and the ability to perform techno-economic analyses of both developing technologies and those that have already matured. Simplified process flow diagrams and an equipment list are the fundamental pieces of information that must be gathered for this procedure. The following are the stages that make up the techno-economic analysis process that was carried out for this thesis:

3.2.3.1 Assumptions

The list of presumptions that were made for the research work that was done for this thesis can be seen in Table 3.1.

Table 3.1: List of Economic Assumptions

| Parameter | Value |
| --- | --- |
| Cost year and currency | 2016 |
| Plant life | 25 years (2-year construction time and 23-year operational lifetime) |
| Interest rate (%) | 7.5 |
| First-of-a-kind or Nth-of-a-kind | Nth-of-a-kind |
| Greenfield or Brownfield | Brownfield |
| Maintenance cost (%) | 4% of the EIC |
| Electricity price (kWh) | 0.12 |
| Cooling water price (m3) | 0.02 |
| Purchased steam price (t) | 17 |
| Solvent (MEA) cost (m3) | 1866 |
| Solvent destruction cost (m3) | 333 |
| Operator cost per person (year) | 77 |
| Engineer cost per person (year) | 150 |
| Operating hours per year | 8000 |
| Currency conversion factor for Year 2016 | 9.5 |
| CO2 capture cost or avoided cost | Capture cost |

3.2.3.2 Simulations

The process is then simulated using Aspen Hysys, Aspen Plus, or another piece of software. The mass and energy balances provided by these simulations are utilized to size the machinery and assess the utility consumption that serves as the basis for the economic analyses. Simulating the procedure is not necessary since this phase may be carried out without the use of computers. The explanation of additional parameters and simulation of CO2 capture plants.

3.2.3.3 Dimensioning of the equipment and its price

Creating a list of process equipment and performing equipment dimensioning, which should include the equipment's size, number of needed pieces, and construction material The key assumptions used while dimensioning the equipment are described. Equipment cost is greatly influenced by the size of the piece of equipment and the materials used in its manufacture. The working circumstances, such as pressure, temperature, kind of fluid, and corrosion risk, affect the material choice.

Table 3.2: Materials for process equipment based on construction materials

| Material of Construction | Material factor (fmat) |
| --- | --- |
| Stainless steel (SS316) welded | 1.75 |
| Stainless steel (SS316) machined | 1.30 |
| Glass-reinforced plastic | 1.0 |
| Exotic materials | 2.50 |

The Aspen In-plant Cost Estimator was used to estimate the cost of the equipment for this thesis. This programme provides the equipment cost based on information gathered from equipment manufacturers rather than using any sort of factorial technique. It is crucial to make sure that the price of the equipment is adjusted for the right size, year, and building material. The majority of materials used in industry, including exotic materials, stainless steel (SS), and carbon steel, often have costs generated from the Aspen programme (CS). Since the installation factor sheet (included in Part 3) used for this approach is based on the cost of carbon steel, if the equipment cost is not for carbon steel, one should utilize material factors (FMAT) to convert to carbon steel using Eq. (3.3).

Equipment CostCS=Equipment CostSS, exotic,../fmat                  (3.3)

3.2.3.4 Improved Computation of the installation factor

Using the installation factor sheet (created by Nils Henrik Eldrup and published in Part 3) for the years 2016–2018, an "improved detailed installation factor" is computed for each piece of equipment. Due to its emphasis on each individual piece of equipment from the time of purchase until it is installed, this technique is known as the Enhanced Detail Factor method. The core of this technique, which covers the direct cost, engineering cost, administration cost, commissioning cost, and contingency cost, is the increased comprehensive installation elements. These variables are calibrated using several constructed plants as well as thorough approximated investigations. Using historical data for a particular site, it is also feasible to calibrate the procedure to that location.

To calculate the installation factor from the installation factor sheet, the following details must be provided:

- Since the installation factor sheet utilizes the Norwegian kroner (NOK) as the currency, equipment costs are calculated using the equivalent cost of carbon steel in NOK. Depending on the region chosen, the cost estimated by the Aspen In-plant Cost Estimator which should be translated into NOK using predetermined exchange rates. If cost escalation is also necessary, the same location's index and currency conversion should be applied.

- It is also necessary to include information regarding the sort of process plant, such as whether it handles solids or fluids.

The sub-factors shown in Figure 3.1 are added together to get the overall installation factor (FTotal,CS). Each piece of equipment will have a different installation consideration. This guarantees that the cost estimate is accurate and reliable, that a whole sub-project is developed around each piece of equipment, and that all the sub-factors needed to determine the cost—from the equipment's foundation to its roof and even its lighting—are taken into account.

Figure 3.1: Main elements - Installation factor

3.2.3.5 EDF technique for calculating the Total Installation Cost

When the material of construction is CS, the equipment installed cost (EIC) for each piece of equipment is estimated using Equations (2) and (3), and when the material of construction is any material other than CS, the EIC is estimated using Equations (4) and (5). The equipment cost is subtracted from the enhanced detailed individual installation factor to arrive at the overall equipment cost. When the material of construction is anything other than CS, the calculation for the installation factor is altered. (6) After that, after the cost of installing each piece of equipment has been determined, the total installed cost (CAPEX) of the whole project may be calculated using the following equation:

EICCSNOK=Equipment CostCS NOK×FTotal, CS                      (3.4)

Where FTotal, CS=fdirect+fengg+fadministration+fcommissioning+fcontingency (3)

EICSS, exoticNOK=Equipment CostCS NOK×FTotal, SS, exotic..            (3.5)

where FTotal, SS, exotic..=[FTotal, CS+fmat-1fequip+fpiping}]           (3.6)

Total Installed Cost Nok= ΣEIC for all equipments                    (3.7)

3.2.3.6 Currency and adjustments

When calculating the overall cost of installation, index regulation and currency regulation are both taken into account as necessary. If the cost that is needed, then the entire cost of the installation using a predetermined conversion rate, as stated in the equation below (3.8).

Total Installed Cost=Total Installed Cost NOK×Exchange rate         (3.8)

If necessary, the cost associated with the location of the plant may be changed.

3.2.3.7 Annualized CAPEX calculation

The annualized component, which is dependent on the interest rate p and plant operating lifespan n, has to be determined first using Eq. (3.9) in order to compute the annualized installed cost, which is also known as the annualized CAPEX. Equation (3.9) is for a building duration of one year and an operating lifespan of twenty-four years. To get the annualized installed cost, or price/yr, divide the total cost of installation by the annualized factor, as shown in the equation below (3.10).

Annualized factor= n=1241(1+p)n                                 (3.9)

Annualized CAPEX= Total installed costAnnualized factor                    (3.10)

3.2.3.8 Calculation of cost of CO2 capture/avoided cost/Cost of steam

The CO2 capture cost in Parts 2 and 3 is computed by dividing the total annualized CAPEX and yearly OPEX (which will be discussed in the next section) by the quantity of CO2 collected or avoided, as shown by the equation. This yields the CO2 capture cost (3.11). Either Aspen simulations or manual calculations may be used to determine the total quantity of CO2 that has been sequestered.

CO2 capture/avoided cost =Annualized CAPEX+Yearly OPEX CurrencyyrAmount of CO2capturedavoidedtyr     (3.11)

When calculating the cost of steam for Part 1 and Part 4, the same formula as Equation (3.11) is used. However, in this case, the annual cost (CAPEX + OPEX) is divided by the total quantity of annual steam output. This formula is found in Equation (3.12).

Steam cost Rston=Yearly cost RsyrSteam produced tonhr*Plant operating hours hrsyear     (3.12)

The Total Plant Costs calculated using the NETL approach depicted in Figure 3.2 are comparable to the Total Installed Costs acquired using the EDF method. According to the AACE categorization scheme, this kind of cost estimate is classified as Class 5 (idea or screening). The EDF cost estimating approach treats each piece of equipment as a separate project, applying distinct installation factors to each piece of equipment in order to maximize the accuracy of the cost estimate. It is crucial to stress that the EDF method does not account for cost escalations, interest accrued during the construction period, costs associated with land preparation and purchase, costs associated with lengthy pipelines and belt conveyors, costs associated with office buildings and workshops, and other expenses incurred by the owner.

Figure 3.2: Levels of capital costs as described in the NETL report

3.2.3.9 Operational and Maintenance Costs

Ordinarily, operational and maintenance expenses (O&M) are split into fixed and variable O&M costs depending on the annual plant operating hours. It is important to note the presumptions used in the OPEX calculations, including those for the number of plant active days per year, the number of operators and engineers needed, and the unit prices for raw materials, solvents, and utilities.

The recurring operating expenses include:

- Maintenance costs

It was decided that 4% of the EIC would go toward the cost of yearly maintenance. This number, which is often reported in the literature to shift anywhere from 2% to 6% based mostly on the kind of plant being studied.

- Operating labor costs

The cost of the yearly operator is calculated based on the number of shift employees, which equals six operators, plus one engineer.

The expenditures associated with are considered part of the variable operating costs:

- Raw materials

- Electricity

- Cooling water

- Steam

- Solvents

- Miscellaneous consumables

Calculation of each of the variable operating expenses described earlier is accomplished by applying the overarching statement presented in Eq (3.13).

Yearly Utility cost Rsyr=Annual consumption Unithr×operating hoursyear×Utility price Rsunit         (3.13)

where unit can be in m3, kg or kWh.

OPEX does not take into account administrative expenses, taxes, insurance, the cost of the initial fill, the costs associated with pre-production, and the costs associated with the transportation and storage of CO2.

3.2.4 Simulations and Specifications

3.2.4.1 Flue gas specifications

Table 3.3 provides an overview of the typical ranges of flow rate and CO2 content in flue gas for the various sectors that were considered for this Master thesis study. Because the same data has been used for several articles, it is also specified which study considered which flue gas in the analysis. This is because multiple parts have utilized the same data.

Table 3.3: Specifications for flue gases utilized in each case study

| Parameter | Cement S1 | Cement S2 [ | Part | Steel | Silicon |
| --- | --- | --- | --- | --- | --- |
| Indication of flue gas | Pre-calciner | Pre-calciner | Recovery | Hot | Electric arc |
|  | String 1 | String 2 | boiler | Stoves | furnace |
| Flow rate (kNm3/hr) | 129.7 | 127.4 | 576.7 | 189.7 | 86.7 |
| Flue gas temperature in for excess heat recovery (oC) | 389 | 382 | 175 | 269 | 600 |
| Flue gas temperature out after heat recovery (oC) | 169 | 169 | 150 | 150 | 150 |
| Flue gas temperature to CO2 capture plant (oC) | 80 | 80 | - | - | - |
| Pressure (kPa) | 101.3 | 101.3 | 101.3 | 101.3 | 101.3 |
| CO2 concentration (mol %) | 22 | 13 | 13 | 25 | 3 |
| Data used in part | Part 1,2, 3 | Part 1, 3 | Part 1 | Part 1 | Part 1 |

#### 3.2.4.2 Simulations of CO2 capture

In Aspen Hysys, a simulation of the post-combustion amine-based CO2 capture plant has been created. The simulation parameters that were used by the Aspen software are listed in Table 3.4. Aspen Hysys is a commercial, all-purpose process simulation software product that was developed by AspenTech. Both the absorption and the desorption columns are modelled using equilibrium stages, and these models take into account stage efficiency (also known as Murphree efficiency). The change that takes place in the mole fraction of CO2 from one stage to the next is what determines the Murphree efficiency for that stage. This change is then divided by the change that would take place if equilibrium were assumed. The following is a specification of the murphree efficiency for CO2 in absorption column stages: The efficiency is set at 0.21 for the first top five stages, and then it declines linearly to 0.11 by the time the process reaches stage 15. In the desorption column, the value of the Murphree efficiency for CO2 has been held constant at 0.5. According to estimates, the efficiencies provided by the Murphree make it possible for each stage to be comparable to one metre of packing height. It is assumed that the pumps and fans have an adiabatic efficiency of 0.75 in the simulation. In order to reach a pressure of 96 bar, the carbon dioxide must first be compressed in four steps. This pressure is then pushed up to 120 bar.

Table 3.4: Simulation parameters for a CO2 capture plant including scenarios for complete and partial capture

| Simulation parameter | Part 3 | Part 2 |
| --- | --- | --- |
| Scenario | Full Capture | Partial Capture |
| Flue gas analyzed | String 1 and 2 | String 1 only |
| Flue gas pre-treatment | Not included | Not included |
| Capture rate | 85 % | 50 % |
| Flue gas temperature from the process | 80°C | 80 °C |
| Inlet flue gas temperature to the absorber | 40°C | 40 °C |
| Inlet gas pressure to the absorber | 1.21 bar | 1.1 bar |
| Inlet flue gas molar flow rate | 11470 kgmol/h | 5788 kgmol/h |
| Lean MEA temperature | 40°C | 40°C |
| Lean MEA pressure | 1.01 bar | 1.01 bar |
| Lean MEA molar flow rate | 96850 kgmole/h | 527500 kg/h |
| MEA content in Lean MEA | 29.0 mass-% | 29.0 mass-% |
| CO2 in Lean MEA | 5.3 mass-% | 5.5 mass-% |
| Number of stages in the absorber | 15 | 15 |
| Murphree efficiency range in the absorber | 0.11–0.21 | 0.11–0.21 |
| stages |  |  |
| Temperature in amine before the desorber | 104.6°C | 101.2 °C |
| Number of stages in the desorber | 10 | 10 |
| Murphree efficiency in the desorber stages | 0.5 | 0.5 |
| Reflux ratio in the desorber | 0.3 | 0.3 |
| Desorber pressure | 2.0 bar | 2.0 bar |
| Reboiler temperature | 120°C | 120 °C |
| Reboiler Power | 117.1 MW | 24.5 MW (only excess heat) |
| ∆Tmin in the lean/rich heat exchanger | 10°C | 10°C |
| CO2 compression | 120 bar (4-stage | Not included |
|  | compression) |  |
| CO2 Transport & Storage | Not included | Not included |

#### 3.2.4.3 Important things to consider regarding equipment Dimensioning

We have made some educated guesses about the size of the process equipment by using standard dimensioning parameters. The velocity that was determined by applying the k-factor of 0.15 metres per second to the Souders-Brown equation served as the basis for the design of the direct contact cooler unit. It is anticipated that the Direct contact cooler (DCC) has a total height of 15 metres, and the packing material that is used in it is stainless steel. The diameter of the absorption column is calculated based on a gas velocity of 2.5 metres per second, while the diameter of the desorption column is calculated based on a gas velocity of 1 metre per second. Each stage of the absorption and desorption columns has a packing height of one metre, and the stage efficiencies have been defined. It is assumed that the total height of the absorption column and the desorption column is 40 metres, and that each column is 22 metres tall. When determining the height of the absorber, it is necessary to take into account the sump, packing, liquid distributors, water wash, demister, gas intake, and gas outlet. The intake for the condenser, packing, liquid distributor, gas inlet, and sump are all factors that are considered while determining the height of the desorber.

The duty and temperature conditions that are received from simulations are used in the calculation process to determine the heat transfer areas of the heat exchangers. For the most part, shell and tube heat exchangers are taken into consideration in this study. The overall heat transfer coefficients that are assumed are as follows: for the lean/rich heat exchanger, 500 W/(m2.K); for the lean amine cooler, 800 W/(m2.K); for the reboiler, 800 W/(m2.K); for the condenser, 1000 W/(m2.K); and for the intercoolers, 800 W/(m2.K In some circumstances, a plate and frame heat exchanger is investigated as a potential alternative to a lean/rich heat exchanger.

For both the rich amine and the lean amine pumps, centrifugal pumps are the preferred choice. In order to compute the cost of the pump's equipment, the volumetric flow rate and pump power that were generated by the simulations are necessary.

3.2.5 Sensitivity Analysis

The costs that have historically been presented in the literature for capture plants frequently do not highlight the design assumptions. It is not possible to optimize the cost without first studying the design parameters, and it is not possible to optimize the cost without studying the design assumptions. The sensitivity analysis of a process is carried out in two different approaches for each case study included in this thesis in order to investigate all of the relevant technical and economic elements.

- A sensitivity analysis is done on the important technical and design factors, such as the steam pressure (and temperature), and the total heat transfer coefficient values for the heat recovery steam generator and condenser are studied for excess heat recovery networks. A sensitivity study of the flue gas flow rate, CO2 capture efficiency, modifying reboiler temperature, and ∆Tmin in the lean/rich heat exchanger is carried out for CO2 capture facilities.

- A sensitivity analysis is performed on the economic parameters and installation variables by taking into account the likelihood of the occurrence of various values. The capital cost, plant lifespan, interest rate, maintenance expenses, steam costs, energy costs, running hours, and total operating costs are the economic factors that have been chosen.

3.3 CO2 CAPTURE PROCESSES COST ESTIMATION USING VARIOUS SOLVENTS/BLENDS

# 3.3.1 Simulations in Aspen Hysys

This section will first go through the processes involved in simulations. The procedures that are used in this study are the standard removal process and the vapour recompression ones. Both of these processes were detailed in the chapter before this one. The study starts with the definition of a basic case in which the absorber is loaded with 30 weight percent MEA as a solvent. The pertinent particulars for this scenario are going to be presented in the following. The work continues with the use of various amounts of additional solvents as well as mixes of those solvents as the solvent. The purpose for this is so that we may have CO2 removal procedures that are improved rather than just the baseline scenario. Also, an ideal mixture that optimizes the removal process in regulating parameters, particularly regeneration energy, will be sought after, and it will be attempted to be found.

Figure 3.3: Typical CO2 removal process plant flowchart

3.3.1.1 Specification of base case simulation

The first step in the simulation is to choose the appropriate attributes for the various components that will be taking part in the process. After that, the package definition for the software has to be completed. The present effort is based on a chemical solvents and acid gas package, which is providing support for amines and the blends of those amines. One of the shortcomings of the chemical solvents and acid gas package used in this study is that it does not include any tri-amine mixes consisting of MEA, MDEA, and PZ. This is one of the limits of the package.

The next step in the process is specifying the pieces of equipment to be used, along with the streams that will enter and exit each component. It is necessary, first and foremost, to establish a practicable base case in order to be able to compare the impacts of using various solvents and their respective mixes. The base case for this work is defined by a previous piece of research titled I L. E., Aspen HYSYS Simulation of CO2 Removal by Amine Absorption from a Gas Based Power Plant, 2007," which describes a scenario in which CO2 is removed from flue gas using an optimized process that contains 30% MEA solvent. In the basic scenario, it has also been anticipated that the lean/rich heat exchanger will have an efficiency of removal of 85% and a minimum approach temperature of 10°C.

Aspen HYSYS versions 10 and 12 have been used to model various carbon dioxide removal processes. The requirements that relate to the basic case are provided in Table 3.5.

Table 3.5: Specifications for the simulation's fundamental CO2 removal mechanism (Aspen HYSYS Simulation of CO2 Removal by Amine Absorption from a Gas Based Power Plant)

| Parameter | Value | Unit |
| --- | --- | --- |
| Inlet flue gas temperature to process | 40 | ℃ |
| Inlet flue gas pressure to process | 101.0 | 𝑘𝑃𝑎 |
| Inlet flue gas flow rate | 1.091e5 | 𝑘𝑔𝑚𝑜𝑙/ℎ |
| CO2 content in inlet gas | 3.30 | 𝑚𝑜𝑙% |
| Water content in inlet gas | 6.90 | 𝑚𝑜𝑙% |
| Lean amine temperature before and after pump | 120 | ℃ |
| Amine pressure before rich pump | 200 | 𝑘𝑃𝑎 |
| Amine pressure after rich pump | 300 | 𝑘𝑃𝑎 |
| Lean amine pressure to absorber | 101 | 𝑘𝑃𝑎 |
| Lean amine rate to absorber | 1.175e5 | 𝑘𝑔𝑚𝑜𝑙/ℎ |
| CO2 content in lean amine | 2.98 | 𝑚𝑜𝑙𝑒% |
| Number of stages in absorber | 10 | - |
| Rich amine pressure before pump | 110 | 𝑘𝑃𝑎 |
| Rich amine pressure after pump | 200 | 𝑘𝑃𝑎 |
| Number of stages of stripper | 6 + Reboiler + Condenser | - |
| Reboiler temperature | 120 | ℃ |
| Efficiency of stages in absorber | 0.25 | - |
| Efficiency of stages in stripper | 1 | - |

The efficiency of stages in the absorber for the base case was assumed to be 0.25 in accordance with Table 3.5; however, the basic scenario was also simulated with the efficiency of stages set at 0.17. The Aspen HYSYS programme made the proposal that this figure be used for the base scenario, although it is impossible for it to be practical.

In addition to the basic scenario, which used MEA at a concentration of 30 weight percent as the solvent, the CO2 removal process was simulated for MEA concentrations of 35 weight percent, 40 weight percent, and 45 weight percent.

The Aspen HYSYS flowsheet for the base case simulation may be seen in the following image.

Figure 3.4: Flowchart of the common CO2 removal procedure from Aspen HYSYS

3.3.1.2 Other solvents and mixes specification

In order to recreate the typical removal process, a number of additional solvents, including MDEA and piperazine (PZ), as well as blends of these solvents, such as MEA plus MDEA and MEA plus PZ, have been used. This has been done in order to simulate the process. The criteria for the base case have been used as the basis for the design of the procedures in many of the cases. The cases that were simulated are broken down and summarized in the following table. In every conceivable situation, the removal efficacy is 85%, and the temperature difference between approaches only varies by a maximum of 10 degrees Celsius.

The following is a list of the seven classifications that each of the simulated standard removal procedures may be placed into:

- Standard removal method using amine blends of MEA and PZ with 30 weight percent amine

- Standard removal technique using amine mixes of MEA and PZ with 40 weight percent amine

- Standard removal technique using amine mixes of MEA and MDEA containing 30 weight percent

- Standard removal technique using amine mixes of MEA and MDEA containing 40 weight percent

- Standard removal technique with amine mixes of MEA and MDEA containing 50 weight percent each

- Standard removal technique using amine mixes of MDEA and PZ containing 40% by weight of amine

- Standard removal technique with amine mixes of MDEA and PZ containing 50 weight percent amine.

Table 3.6: Process simulations for the conventional procedure using different solvents and mixes

| 30 wt% MEA+PZ | 40 wt% MEA+PZ | 30 wt% MEA+MDEA | 40 wt% MEA+MDEA | 50 wt% MEA+MDEA | 40 wt% MDEA+PZ | 50 wt% MDEA+PZ |
| --- | --- | --- | --- | --- | --- | --- |
| 30% MEA | 40% MEA | 30% MEA | 40% MEA | 50% MEA | 40% MDEA | 50% MDEA |
| 27.5% MEA+2.5% PZ | 35% MEA+5% PZ | 25% MEA+5% MDEA | 35% MEA+5% MDEA | 45% MEA+5%MDEA | 35% MDEA+5% PZ | 45% MDEA+5%PZ |
| 25% MEA+5% PZ | 30% MEA+10% PZ | 22.5% MEA+7.5% MDEA | 30% MEA+10% MDEA | 40%MEA+10%M DEA | 30% MDEA+10% PZ | 40%MDEA+10% PZ |
| 22.5% MEA+7.5% PZ | 25% MEA+15% PZ | 20% MEA+10% MDEA | 25% MEA+15% MDEA | 35%MEA+15%M DEA | 25% MDEA+15% PZ | 35%MDEA+15% PZ |
| 20% MEA+10% PZ | 20% MEA+20% PZ | 15% MEA+15% MDEA | 20% MEA+20% MDEA | 30%MEA+20%M DEA | 20% MDEA+20% PZ | 30%MDEA+20% PZ |
| 15% MEA+15% PZ | 15% MEA+25% PZ | 10% MEA+20% MDEA | 15% MEA+25% MDEA | 25%MEA+25%M DEA | 15% MDEA+25% PZ | 25%MDEA+25% PZ |
| 10% MEA+20% PZ | 10% MEA+30% PZ | 5% MEA+25% MDEA | 10% MEA+30% MDEA | 20%MEA+30%M DEA | 10% MDEA+30% PZ | 20%MDEA+30% PZ |
| 5% MEA+25% PZ | 5% MEA+35% PZ | 30% MDEA | 5% MEA+35% MDEA | 15%MEA+35%M DEA | 5% MDEA+35% PZ | 15%MDEA+35% PZ |
| 30% PZ | 40% PZ |  | 40% MDEA | 10%MEA+40%M DEA | 40% PZ | 10%MDEA+40% PZ |
|  |  |  |  | 5%MEA+45%MD EA |  | 5%MDEA+45%P Z |
|  |  |  |  | 50%MDEA |  | 50%PZ |

The above list includes all of the simulated standard procedures. Of every scenario, it was assumed that the efficiency of the stage in the absorber was 0.25. In addition to table 3.6, two further instances were simulated, one of which had a mix of 30 weight percent MEA and 5 weight percent PZ with the identical parameters as the tabulated examples with ME values of 0.3 and 0.35.

3.3.1.3 Specifications for the process of vapour compression

Figure 3.5: Process for removing CO2 from vapour recompression flowsheet developed by Aspen

The vapour recompression process was modelled using a MEA solvent of 30 weight percent. It was expected that the removal efficiency for the simulated situation would be 85%, and the lean minimum approach temperature would be set at 10°C. Additionally, it was assumed that now the efficiency of each level in the absorber was 0.25.

The requirements and standards for the process of vapour recompression are, for the most part, the same. The addition of a valve, a separator, and a compressor to the normal procedure is the most significant change brought about by this study. The pressure of the stripper's output stream is lowered from 200 kPa to 100 kPa as a result of the valve, which corresponds to the changes in the process parameters. A linear valve with a 50% opening is responsible for bringing about this decrease in pressure. The separator is operating at a pressure of 100 kPa. At a pressure of one hundred kilopascals, the liquefied portion of the inflow stream into the separator emerges out the bottom of the separator item. The vapour exits the separator at a pressure of one hundred kilopascals (kPa). With the assistance of the lean pump and compressor, respectively, the pressure of the liquid and gas will be brought up to the appropriate levels. In point of fact, the pressure of the liquid has to be brought up to 300 kPa, while the pressure of the vapour needs to be brought up to 200 kPa.

In addition to what was mentioned above, a simulation of the vapour recompression process was carried out for amine mixes consisting of 30% MEA and 15% MDEA, as well as 30% MEA and 5% PZ. Bott noted that mixes have the same parameters as the simulation given before in terms of LMTD, removal efficiency, and the efficiency of the stages in the absorber.

Also, vapour recompression for a combination of 30 weight percent MEA and 15 weight percent MDEA was simulated in a lean-rich heat exchanger at a temperature of 5 degrees Celsius.

3.3.1.4 Simulation results

Every simulation that is discussed in this chapter was performed using Aspen HYSYS version 10. We have looked at the simulated instances in terms of regeneration energy, lean amine loading, rich amine loading, and cyclic loading. The extracted findings will have an explanation provided, which is devoted to the discussion of the outcomes.

The primary goal of the extensive number of simulations that are a part of this study is to identify those mixtures of amines that provide lower regeneration energy. This will allow overall costs for removal plants to be reduced, in addition to other improvements.

3.3.2 Dimensioning and Equipment

First and foremost, this section consists of dimensioning, which explains the sizes and materials of the main pieces of equipment used in the CO2 removal process. In addition to this, the criteria that were used to pick each individual component of the plant will be discussed. In addition to that, the applicable equations for each piece of apparatus are provided in this chapter.

The data that is extracted from dimensioning provides the necessary data for the economic analysis of the project. To put it another way, the Aspen In-Plant Cost Estimator makes use of the data in order to compute the overall cost of operating the simulated plant.

The following provides a concise explanation of each component found in the plant.

3.3.2.1 Absorption column

The transfer of species from the gaseous phase to a liquid solvent is what we mean when we talk about absorption, which is the most critical step in a CO2 removal plant. Gas absorption, gas scrubbing, and gas washing are some of the other names that have been used to refer to this procedure. This method is used in the separation of gas mixtures, the recovery of compounds, and the elimination of contaminants (Seader & Henley, 2006).

The flue gas and liquid solvent flow in a counter-current direction throughout the absorption process. In other words, flue gas, which may include carbon dioxide, enters the column from the bottom, while solvent enters the column from the top. Within the column, contact devices are inserted in order to give the highest possible surface area for interaction between the flue gas and the liquid solvent (Kallevik, 2010).

There are several variants of the absorption column to choose from. In this particular piece of work, a packaging kind is used. The table that follows presents the required specs for the absorber that will be used in this project.

Table 3.7: Details on the absorption column

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Column Type | - | - | Packed | Specified |
| Gas velocity | vgas | [𝑚⁄𝑠] | 2.5 | Specified |
| Actual Volume flow rate | 𝑉̇ | [𝑚3⁄𝑠] | Extracted from simulation | Specified |
| Number of stages | Nstages | - | 10 | Specified |
| Height of Packing | hpacking | [𝑚⁄𝑠𝑡𝑎𝑔𝑒] | 1 | Specified |
| Volume of packing | Vpacking | [𝑚3] | Based on formula 3.15 | Calculated |
| Column height | 𝐻 | [𝑚] | 40 | Estimated |
| Column Diameter | 𝐷 | [𝑚] | 19.4 | Calculated |
| Shell Material | - | - | SS316 | Specified |
| Packing Type | - | - | Structured | Specified |

Using the formula 3.14 that is provided below, one may get the value of the absorption column. In addition to the packing height, the column should have room for a bottom liquid reservoir as well as a water wash column. Because of this, (Kallevik, 2010) and Øi L. E., Removal of CO2 from exhaust gas) proposed that an additional height be added to the column in order to give enough area for the input, top and bottom outputs, and auxiliary equipment.

The height of the packing section is determined by the total number of equilibrium stages present in the column. It should be brought to everyone's attention that according to Øi L. E., Removal of CO2 from exhaust gas), one stage is presumed to be equivalent to one metre of height. In order to calculate the overall packing height, then, the number of steps must be multiplied by one metre.

(Ali, Eldrup, Normann, Skagestad, and I Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) Øi L. E., Removal of CO2 from exhaust gas) proposed that the intake gas velocity to the absorption column is 2.5 [ms] per second. Because of this assumption, the formula for estimating the diameter of the column is 3.14.

D=4.Vπ.vgas                            (3.14)

Where, 𝐷 (𝑚) indicates calculated diameter of absorption column, Vm3/s while vgas (m/s) is the expected intake gas velocity to the absorber, is the actual gas volumetric flow rate that is being delivered to the column.

The actual gas volumetric flow rate is fairly high within the scope of this research, which results in a column diameter of around 20 metres. This is due to the fact that the study is examining the flow of real gas. Therefore, the principal part of the column is broken into many smaller sections that have packing capacities that are equivalent to those of the primary section.

For the purpose of calculating the overall volume of the packing, the formula 3.15 is used.

Vpacking=π.D24. Nstages                      (3.15)

Vpacking (𝑚3) is the amount of space used by the packing in the absorber, D (m) is the diameter that was determined, and Nstages represents the total number of steps in the column.

According to Arachchige and Melaaen (2012), there are two distinct types of packing materials that may be used for gas absorption, and these are structured packing and random packing. Figure 3.6 below presents an example of structured packing. Because of its greater efficiency, larger capacity, and lower pressure drop throughout the column, structured packing is preferable to random packing in large-scale CO2 absorption columns (Øi. L. E., Removal of CO2 from exhaust gas) (Brickett, 2015). It is clear from looking at figure 3.6 that structured packing has the ability to provide the removal process a greater contact area between gas and liquid, which would result in a higher total mass transfer coefficient.

Figure 3.6: Structured (right) and Random (left) packing

3.3.2.2 Desorption column

In order to extract CO2 from solution using a desorption column, heat is required. The amine that has been regenerated enters the cyclic removal process from the bottom of the column, while recovered carbon dioxide exits the column at the top. The condenser is located at the top of the column, while the reboiler is located at the bottom. There are a lot of parallels to be seen between the construction and operation of an absorption column and this column. Calculating the dimensions of this piece of machinery using formulas 3.14 and 3.15 is also part of the process. According to (Ali, Eldrup, Normann, Skagestad, & Øi Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) and Øi L. E., Removal of CO2 from exhaust gas), an entrance velocity of flow to column of 1.0 ms is a good choice.

The column may be constructed according to the parameters listed in table 3.8

Table 3.8: Desorption column specifications

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Column Type | - | - | Packed | Specified |
| Flow velocity to column | vgas | [𝑚⁄𝑠] | 1 | Specified |
| Actual volume flow rate | 𝑉̇ | [𝐴 m3/s] | Table K.2 | HYSYS |
| Number of stages | Nstages | - | 6 + reboiler + condenser | Specified |
| Height of packing | hpacking | [𝑚⁄𝑠𝑡𝑎𝑔𝑒] | 1 | Specified |
| Volume of packing | Vpacking | [𝑚3] | Based on formula 3.15 | Calculated |
| Column height | 𝐻 | [𝑚] | 20 | Assumed |
| Column Diameter | 𝐷 | [𝑚] | Based on formula 3.14 | Calculated |
| Shell Material | - | - | SS316 | Specified |
| Packing Type | - | - | Structured | Specified |

## 3.3.2.3 Lean / Rich Heat Exchanger

When it comes to the removal process, utilizing a lean rich heat exchanger is done with the intention of recovering heat. In other words, the rich amine that is located at the bottom of the absorber is heated up by a hot solvent, which is the lean amine that is located in the sump of the stripper. An increase in the rich amine temperature results in a decreased need for reboiling (Kallevik, 2010).

In order to accomplish the objectives of this study and fulfil the requirements, a heat exchanger of the shell and tube type, as illustrated in figure 3.7, is used. According to Aromada, Eldrup, Normann, and Øi (2020), this particular kind of heat exchanger is the most durable.

Figure 3.7: Shell and tube heat exchanger

Formula 3.16 is used to calculate the size of the lean rich heat exchanger. Other requirements for lean rich heat exchangers may be found in table 3.16, which can be seen below.

A.QU.∆Tlm                    (3.16)

where, 𝐴 (𝑚2) reveals the heat exchanger's overall area, 𝑄̇ (𝑘𝑊) is heat exchanger duty and U (W/m2K)

U (W/m2K) is the global coefficient of heat transfer. Equation 3.17 computes ∆Tlm, the following, using a logarithmic mean temperature difference.

∆Tlm=∆Tout-∆Tinln∆Tout∆Tin                          (3.17)

Where, ∆Tlm (℃) shows temperature using a logarithmic minimum technique, ∆Tout (℃) is referred to as the differential between the heat exchanger's cold and hot entrance streams, (∆Thot,in, − ∆Tcold,in), and ∆Tln (℃) the distinction between hot and cold outlet streams, (∆Thot,in, − ∆Tcold,in).

It is important to note that the total heat transfer coefficient, U, for lean rich heat exchanger is assumed to be 500 (W/m2K) in Ali, Eldrup, Normann, Skagestad, and i's 2019-part, Cost Estimation of CO2 Absorption Plants for CO2 Mitigation - Method and Assumptions.

The table that follows is a listing of the pertinent specs for the lean rich heat exchanger.

Table 3.9: Lean-rich heat exchanger specifications

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Heat Exchanger Type | - | - | U-Tube Flow | Specified |
| TEMA type | - | - | BEU | Specified |
| Heat transfer rate / Duty | 𝑄̇ | [𝑘𝑊] | Based on simulations | HYSYS |
| Overall heat transfer coefficient | 𝑈 | [𝑊⁄𝑚2𝐾] | 500 (Ali, Eldrup, Normann, Skagestad, & Øi, Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) | Specified |
| Temperature of hot flow into the heat exchanger | Thot,in | [℃] | Based on simulations | HYSYS |
| Temperature of hot flow out of heat exchanger | Thot,out | [℃] | Based on simulations | HYSYS |
| Temperature of cold flow into heat exchanger | Tcold,in | [℃] | Based on simulations | HYSYS |
| Temperature of cold flow out of heat exchanger | Tcold,out | [℃] | Based on simulations | HYSYS |
| Logarithmic mean temperature difference | ∆Tlm | [℃] | Based on formula 3.17 | HYSYS/Calculated |
| Total area required | 𝐴 | [𝑚2] | Based on formula 3.16 | Calculated |
| Shell material | - | - | SS316 | Specified |
| Tube Material | - | - | 316LW | Specified |

The estimated overall area for a lean-to-rich heat exchanger in this work is a very significant amount of space. Because of this, it is separated into many more manageable parts. The assumption used for this division is that the most common size of shell on the market is 1000 m2. Therefore, the total number of heat exchangers that are less capacious is computed.

A scaling factor of 0.65 has to be employed in each and every instance when estimating the cost of the overall smaller heat exchanger.

3.3.2.4 Reboiler

A reboiler is a kind of heat exchanger that is positioned at the base of a desorption column. Its purpose is to bring the liquid to a boil and produce vapour. The majority of reboilers are heat exchangers with shells and tubes, and steam is often utilized to deliver the necessary amount of heat. The dimensions of the lean rich heat exchanger and the reboiler are entirely comparable to one another. Consequently, the most current formulae offered, compute the entire area for the reboiler

The next table, 3.10, provides further parameters for the reboiler that will be used in the project.

(Ali, Eldrup, Normann, Skagestad, and Øi Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) The overall heat transfer coefficient, U, for the Reboiler is assumed to be 800 (W/m2K).

Table 3.10: Reboiler Specification

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Heat Exchanger type | - | - | U-Tube Flow | Specified |
| TEMA type | - | - | BEU | Specified |
| Heat transfer rate / Duty | 𝑄̇ | [𝑘𝑊] | Based on simulations | HYSYS |
| Overall heat transfer coefficient | 𝑈 | [Wm2k] | 800 (Ali, Eldrup, Normann, Skagestad, & Øi, Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) | Specified |
| Temperature of steam to heat exchanger | Thot,in | [℃] | 131 | Assumed |
| Temperature of steam hot from heat exchanger | Thot,out | [℃] | 130 | Assumed |
| Temperature of cold flow into heat exchanger | Tcold,in | [℃] | Based on simulations | HYSYS |
| Temperature of cold flow out of heat exchanger | Tcold,out | [℃] | Based on simulations | HYSYS |
| Logarithmic mean temperature difference | ∆Tlm | [℃] | Formula 3.17 | Calculated |
| Total area required | 𝐴 | [𝑚2] | Formula 3.16 | Calculated |
| Shell material | - | - | SS316 | Specified |
| Tube material | - | - | 316LW | Specified |

## 3.3.2.5 Condenser

A condenser is a special kind of heat exchanger that allows the gaseous phase of a material to transition into the liquid phase when it is cooled down. The transfer of heat to another material is required for the process of condensation. Because of this, water that has been cooled down is utilised to remove heat from the gaseous stream.

When it comes to determining the size of the condenser, the appropriate formulas for the reboiler and the lean rich heat exchanger may be used. Therefore, formulae 3.16 and 3.17 are used to determine the needed amount of heat transfer area for the condenser. (Ali, Eldrup, Normann, Skagestad, and Øi Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) The overall heat transfer coefficient, U, for the condenser is assumed to be 1000 (W/m2K).

The condenser's full range of characteristics and specifications are detailed in Table 3.11.

Table 3.11: Condenser Specification

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Heat Exchanger type | - | - | U-Tube Flow | Specified |
| TEMA type | - | - | BEU | Specified |
| Heat transfer rate / Duty | 𝑄̇ | [𝑘𝑊] | Based on simulations | HYSYS |
| Overall heat transfer coefficient | 𝑈 | [𝑊⁄𝑚2𝐾] | 1000 (Ali, Eldrup, Normann, Skagestad, & Øi, Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) | Specified |
| Temperature of inlet hot flow | Thot,in | [℃] | - | HYSYS |
| Temperature of outlet hot flow | Thot,out | [℃] | - | HYSYS |
| Temperature of inlet cooling water | Tcold,in | [℃] | 8 | Assumed |
| Temperature of outlet cooling water | Tcold,out | [℃] | 23 | Assumed |
| Logarithmic mean temperature difference | ∆Tlm | [℃] | - | Calculated |
| Total area required | 𝐴 | [𝑚2] | - | Calculated |
| Shell material | - | - | SS316 | Specified |
| Tube material | - | - | 316LW | Specified |

## 3.3.2.6 Lean amine cooler

A form of heat exchanger known as a lean amine cooler is responsible for bringing the temperature of a stream of lean amine down to an optimal level with the assistance of cooling water. Because of the significant impact that this temperature has on the overall effectiveness of the removal process, the temperature of the lean amine has to be adjusted to an optimal value. It has been assumed in a great number of investigations that the optimal value for the entrance temperature of lean amine to absorber is forty degrees Celsius. (Abu-Zahra, Schneiders, Niederer, Feron, and Versteeg, 2007) investigated the effect of lean amine temperature on the absorber and suggested that there ought to be a trade-off between the required amount of cooling water in the lean amine cooler and the required amount of regeneration energy in the stripper. They performed a simulation of a 90% amine-based removal process using 30% MEA as the solvent, and they discovered that increasing the temperature of lean amine before it is introduced to the absorber results in an increase in the amount of regeneration energy required for the process, as well as a decrease in the amount of cooling duty required for the lean amine cooler. On the other hand, a lower temperature of lean amine being introduced to the absorber results in less energy being regenerated but a greater need for cooling. As a result, the temperature will need to be optimized.

Figure 3.8: For a 90% MEA-based removal procedure, evaluation of lean amine temperature and regeneration energy (left) and cooling water (right)

Formulas 3.16 and 3.17 are used to compute the total area of the lean amine cooler that is needed. (Ali, Eldrup, Normann, Skagestad, and Øi Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) The overall heat transfer coefficient for the lean amine cooler is assumed to be 800 (W/m2K).

Dimensioning needs for the lean amine cooler are detailed in Table 3.12, which provides essential parameters and specifications.

Table 3.12: Lean Amine Cooler Specifications

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Item type | - | - | U-Tube Flow | Specified |
| TEMA type | - | - | BEU | Specified |
| Heat transfer duty | 𝑄̇ | [𝑘𝑊] | Based on simulations | HYSYS |
| Overall heat transfer coefficient | 𝑈 | [𝑊⁄𝑚2𝐾] | 800 (Ali, Eldrup, Normann, Skagestad, & Øi, Cost Estimation | Specified |
|  |  |  | of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions, 2019) |  |
| Temperature of inlet hot flow | Thot,in | [℃] | Based on simulations | HYSYS |
| Temperature of outlet hot flow | Thot,out | [℃] | 40℃ | Specified |
| Temperature of inlet cooling water | Tcold,in | [℃] | 8 | Specified |
| Temperature of outlet cooling water | Tcold,out | [℃] | 23 | Specified |
| Logarithmic mean temperature difference | ∆Tlm | [℃] | Formula 3.17 | Calculated |
| Total area required | 𝐴 | [𝑚2] | Formula 3.16 | Calculated |
| Shell material | - | - | SS316 | Specified |
| Tube material | - | - | 316LW | Specified |

## 3.3.2.7 Pumps

Pumps are mechanical devices that are used to move fluids from one location to another by supplying the necessary energy at the transfer point. Pumps may often be categorized in a limitless number of ways, depending on their size, kind, or usage.

The amine-based CO2 collection method typically calls for the use of two pumps, referred to respectively as the rich pump and the lean pump. The rich amine flow is transferred from the sump of the absorber to the top of the stripper by the rich pump. In addition, the lean pump is responsible for supplying the necessary energy to move the flow of lean amine from the base of the desorption column to the top of the absorption column. Overall, these pumps should be able to produce enough energy to make up for a variety of losses, including:

- Losses caused by friction in the pipes

- The loss of pressure in the lean rich heat exchanger that is located between the two columns.

- The difference in pressure between the absorption column and the desorption column

- A decrease in pressure in the lean amine cooler.

For the sake of this study, it has been assumed that the rich pump and the lean pump are both centrifugal pumps with an adiabatic efficiency of 75%.

The needed power, fluid head, and volumetric flow rate of fluid may all be retrieved from simulations and used in the dimensioning process of a pump. It is believed that the fluid head for rich pumps is 50 (𝑚), whereas the fluid head for lean pumps is 70 (𝑚).

The necessary parameters and specifications for Rich and Lean Pumps are outlined in the following table.

Table 3.13: Rich and lean pumps Specification

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Item type | - | - | Centrifugal | Specified |
| Driver type | - | - | Motor | Specified |
| Driver power | 𝑃 | [𝑘𝑊] | Based on simulations | HYSYS |
| Volumetric flow rate | V | [𝑙⁄𝑠] | Based on simulations | HYSYS |
| Pump material | - | - | SS316 | Specified |
| Pump adiabatic efficiency | - | % | 75 | Specified |
| Fluid head for lean amine | ∆Hlean pump | [𝑚] | 70 | Specified |
| Fluid head for rich amine | ∆Hrich pump | [𝑚] | 50 | Specified |

## 3.3.2.8 Fan & Compressor

Fans and compressors are examples of the kinds of turbomachinery equipment that are used to augment the driving power of a fluid by the addition of energy.

A fan is included in the CO2 removal process so that the requisite driving power of flue gas may be supplied to counteract the pressure drop that occurs in the absorption column. For the sake of this study, an adiabatic efficiency of 75% was assumed for the centrifugal fan.

In the vapour recompression process, in addition to the fan, a compressor is also necessary to raise the pressure of flow caused by stripping pressure. The stripper pressure in this investigation is set at a value of 200 (kPa). This goal may be accomplished by using a centrifugal compressor that has an adiabatic efficiency of 75%.

The following is a list of pertinent characteristics and standards for the dimensioning of the fan.

Table 3.14: Fan Specification

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Item type | - | - | Centrifugal | Specified |
| Driver type | - | - | Motor | Specified |
| Driver power | 𝑃 | [𝑘𝑊] | Based on simulations | HYSYS |
| Actual volumetric flow rate | V | [𝑚3⁄ℎ𝑟] | Based on simulations | HYSYS |
| Material | - | - | CS | Specified |
| Adiabatic efficiency | - | % | 75 | Specified |
| Speed | - | [𝑟𝑝𝑚] | 1800 | Specified |

The necessary specifications for the compressor to be used in the vapour recompression process are listed in Table 3.15.

Table 3.15: Compressor Specification

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Item type | - | - | Centrifugal | Specified |
| Driver type | - | - | Motor | Specified |
| Driver power | 𝑃 | [𝑘𝑊] | Based on simulations | HYSYS |
| Actual volumetric flow rate | 𝑉̇ | [𝑚3⁄ℎ𝑟] | Based on simulations | HYSYS |
| Material | - | - | CS | Specified |
| Design pressure inlet | - | [𝑘𝑃𝑎] | Based on simulations | HYSYS |
| Design pressure outlet | - | [𝑘𝑃𝑎] | Based on simulations | HYSYS |

Because of the high flow rate of flue gas in this job, the size of the fan is significantly increased. It is thus presumed that there are two fans operating in order to provide the necessary driving power for the procedure. For the sake of this division, we will assume that the scaling factor is 1.

3.3.2.9 Separator

The vapour recompression process needs a separator item in order to separate the two-phase flow that is exiting the desorption column, as was explained before. The use of a vertical divider accomplishes this goal well. The fact that liquid is the predominant phase of flow was the primary factor in deciding to go with the vertical one. The Sounder – Browns (Ks) method is used for the dimensioning of the separator, and the corresponding equation is: (Moshfeghian, 2015).

VGmax=KsρL-ρGρG                                 (3.18)

where, VGmax (m/s) specifies the maximum gas velocity that is allowed, KS (m/s) defines as sizing parameter, ρL(kg/m3) demonstrates the density of the liquid phase and ρG(kg/m3) refers to the density of the flow's gaseous phase.

It is safe to presume that the recommended value of KS is 0.081. Additionally, the density of the stream's liquid and gas phases is determined by simulations of their operations. Therefore, in accordance with formula 3.18, VGmax is computed, which, when combined with formula 4.6, yields the separator's diameter.

Dmin=(4/π)qa(FgVGmax)                                        (3.19)

where, 𝐷𝑚𝑖𝑛 (𝑚) is the smallest diameter that must be satisfied for the separator, qa (m3/s) is the gas flow rate at the condition where it is really flowing, and is the proportion of the cross-section area that is accessible for gas flow; for a vertical separator, it is assumed that this term is 1 (Moshfeghian, 2015). (Orangi , Farsi Madan, Fajferek, Saeter, & Bahri, 2020). It is possible to determine the height of the separator based on a predetermined ratio of (Moshfeghian, 2015)

LDmin=2.5                                  (3.20)

where the height of the application is denoted by the unit 𝐿 (𝑚).

Below you will find a list that contains all of the necessary separator criteria.

Table 3.16: Separator item Specifications

| Parameter | Symbol | Unit | Specification | Note |
| --- | --- | --- | --- | --- |
| Separator orientation | - | - | Vertical | Specified |
| Diameter | 𝐷𝑚𝑖𝑛 | [𝑚] | Based on 3.19 | Calculated |
| Length | 𝐿 | [𝑚] | Based on 3.20 | Calculated |
| Material | - | - | SS316 | Specified |

## 3.3.2.10 Non-listed equipment

Only the most important pieces of equipment are described here since the purpose of this work is to do early research on a CO2 removal process using amines other than MEA and their mixes. To put it another way, some components, such as a direct contact cooler (DCC), mixer, and control valve, are not a part of this operation.

The next step in this process is to estimate the costs. Therefore, in order to have more accurate thoughts based on the economy of processes, twenty percent of the total cost of each process is added to the total cost that was calculated to cover non-listed equipment.

# 3.3.3 Economy of Project

First and foremost, this portion of the work comprises explanations that are essential for cost estimate methodologies, as well as the contents of CAPEX and OPEX. The performed cost estimate for simulated processes is the next topic covered in this chapter. The feasibility of the project has been calculated using data taken from the most recent version of the Aspen In-Plant Cost Estimator. Cost estimates for a variety of items of equipment for the year 2016 are included in this programme.

3.3.3.1 OPEX and CAPEX

In general, expenditures may be broken down into two distinct categories: operational expenditure and capital expenditure, sometimes known as CAPEX and OPEX respectively (OPEX). In point of fact, the term "CAPEX" refers to one-time payments made for the purpose of obtaining services, as well as the purchase of tangible assets and items. Due to the fact that they have an effect on the production capacity of the company over the course of an extended period of time (Dikov, 2020), many businesses see CAPEX as a kind of significant investment. One further reason for their significance is that they have rather high starting prices.

(Kallevik, 2010) divided CAPEX into the following categories:

- Property acquisition

- Ground preparation

- Utility connections to nearby infrastructure

- Administrative buildings, rooms, and offices.

CAPEX does not include funding for ground and buildings in this work.

OPEX, which stands for operational costs or expenditure, on the other hand, refers to the continuing cost of running a product, service, or system. This category also includes relevant money for system maintenance. Workers' salary and pay, taxes, raw materials, replacement parts, and rent costs are all instances of OPEX (Kallevik, 2010).

3.3.3.2 CAPEX

The CAPEX calculations for this job begin with estimating the cost of each component in the plant that is sized and defined in the dimension section. There are many methods for estimating or obtaining the cost of industrial equipment. Different sources used in capital cost estimations include quoted offer from vendor, budgeted prices, in-house data from other projects, commercial databases, books, and the Internet (Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation).

It is extremely preferred and dependable to have access to the most recent data for each item in the plant from the manufacturer, but this is not always attainable. (Cost Estimation of CO2 Absorbtion Plants for CO2) recommended that using in-house data from prior projects is a useful way to analyses the project's economics.

Commercial databases, as previously indicated, are also accessible and may be utilized to estimate equipment costs. CAPEX in this project is calculated by extracting equipment costs using Aspen In-Plant Cost Estimator version 10. This version's prices are for the year 2016.

Specifications for each component of the removal plant are critical for obtaining CAPEX since they give necessary data for the Aspen In-Plant Cost Estimator. The cost of each item is affected by the size of each item, the number of needed components, and the materials used in the building process, as outlined in the preceding chapter. Material selection is entirely dependent on operating parameters such as pressure, temperature, corrosion, and fluid type (Ali, Eldrup, Normann, Skagestad, & Øi Cost Estimation of CO2 Absorption Plants for CO2 Mitigation - Method and Assumptions, 2019).

3.3.3.3 Techniques of Enhanced detailed factor (EDF)

Data extracted from the Aspen In-Plant Cost Estimator show equipment expenses, however CAPEX is not restricted to this. Other expenditures must be included, such as direct charges, engineering costs, administrative costs, and commissioning and contingency costs. Each of the terms given contains subcategories. For example, only direct costs are allocated to more specific parts such as erection, plumbing, electric, instrumentation, civil work, steel & concrete, and insulation. These are mentioned below.

Table 3.17: Main elements constituting installing factor

| Direct costs | Engineering costs | Administration costs | Other costs |
| --- | --- | --- | --- |
| Equipment | Process | Procurement | Commissioning |
| Erection | Mechanical | Project control | Contingency |
| Piping | Piping | Site management |  |
| Electric | Electric | Project management |  |
| Instrument | Instrument |  |  |
| Civil work | Civil |  |  |
| Steel & concrete | Steel & concrete |  |  |
| Insulation | Insulation |  |  |

As a result, the extracted equipment costs from the Aspen In-Plant Cost estimator must be modified by some coefficient in order to incorporate all of the important elements in table 3.17. Following that, the results show the total installation cost for each piece of equipment.

3.3.3.4 Material factor

As previously stated, each item in the removal plant is assigned to a certain kind of material. Stainless steel and carbon steel are employed in this job. As a result, the material factor should be introduced for each item. The rationale for this modification is because the EDF table is made of CS material.

The material elements for each category are described below.

Table 3.18: Material considerations for various types of building

| Sort of Material | Material Factor |
| --- | --- |
| Stainless steel (SS316) welded | 1.75 |
| Stainless steel (SS316) machined | 1.30 |
| Glass-reinforced plastic | 1.0 |
| Exotic materials | 2.50 |

A deeper look at table 3.18 above reveals that stainless steel is classified into two types: welded and machined. Welded stainless steel elements in the CO2 capture removal facility.

- Rich amine pump

- Lean amine pump

while machined ones include:

    - Absorber

    - Stripper

    - Lean Rich Heat Exchanger

    - Lean Amine Cooler

    - Condenser

    - Reboiler

The material and material factor specifications for any piece of equipment are provided in the table below. This table is based on the publication (Ali, Eldrup, Normann, Skagestad, & Øi Cost Estimation of CO2 Absorption Plants for CO2 Mitigation - Method and Assumptions, 2019).

Table 3.19: Material specifications and how they affect equipment

| List of equipment | Material | Material factor |
| --- | --- | --- |
| Absorber | SS316 | 1.75 |
| Stripper | SS316 | 1.75 |
| Lean Rich Heat Exchanger | SS316 | 1.75 |
| Lean Amine Cooler | SS316 | 1.75 |
| Condenser | SS316 | 1.75 |
| Reboiler | SS316 | 1.75 |
| Fan | CS | 1.0 |
| Rich amine pump | SS316 | 1.3 |
| Lean amine pump | SS316 | 1.3 |
| Compressor | CS | 1 |
| Separator | SS316 | 1.75 |

To properly adapt the material to the EDF table, the cost of the equipment associated with each individual item has to be split by the material factor. This table may be found towards the conclusion of the appendix. So,

equipment costCS=equipment costcostfmat                    (3.21)

where, fmat may be taken from table 3.19, and the value that is obtained after doing so can, at the moment, be used to locate an acceptable total installed cost coefficient within the EDF table for the specific item in question. This coefficient may be used for the material known as carbon steel. Therefore, the coefficient has to be transformed into a form that is applicable to SS material. Formula 3.22 is the tool that is used for this assignment.

ft,SS=ft,CS+fm,SS-fm,CS+fp,SS-fp,CS                    (3.22)

where ft,SS represents the total cost factor of stainless steel and ft,CS represents the total cost factor of carbon steel, fm,SS represents the material cost factor of stainless steel, and fm,CS represents the cost factor of carbon steel. Finally, fp,SS represents the piping cost factor of stainless steel and fp,CS represents the cost factor of carbon steel.

The total cost of installation for each piece of equipment may be determined by taking the equipment cost from the Aspen In-Plant Cost Estimator and multiplying it by the computed factor. The total CAPEX is the sum of the CAPEX for each individual component.

3.3.3.5 Cost Index for Chemical Engineering Plants

The prices of the equipment that are provided in section 5.4 pertain to the year 2016, while the project in question will be carried out in the year 2022. Due to the fact that the value of money might fluctuate from one period to the next, it is necessary to make adjustments to the costs. The chemical engineering plant cost index, also known as CEPCI, is a tool that can be used to update the capital cost of a chemical plant from an earlier time slot to a later one. This objective may be accomplished with its help.

Since Aspen In-Plant Cost Estimator version 10 only contains data for 2016, and the present project won't be finished until 2021, the CEPCI figures for 2016 and 2021 should be combined to get an accurate CAPEX estimate.

Table 3.20: Chemical Engineering Plant Cost Index

| Year | Chemical Engineering Plant Cost index (CEPCI) |
| --- | --- |
| 2016 | 542 |
| 2021 | 655.7 |

This adjustment is done with

Cost2021=Cost2016×CEPCI2021 CEPCI2016                     (3.23)

3.3.3.6 Power law

The concept of power law refers to a functional connection in which one quantity is calculated based on its proportionate relationship with another variable. This relationship can be expressed mathematically as 3.24, where Cost1 represents the calculated cost for equipment with capacity Q1, Cost2 represents the intended cost for equipment with capacity Q2, Q1 represents the capacity of item 1, Q2 represents the capacity for item 2, and 𝑒 is referred to as the scaling constant or cost exponent (Orangi, Farsi Madan, Fajferek, Saeter, & Bahri, 2020).

As a further explanation, if the cost estimate of an absorber column with capacity Q1 is Cost1, then the cost of an absorber column that is functionally identical but has a capacity that is different from Q2 will be connected in the manner that:

Cost2=Cost1Q2Q1e                            (3.24)

The scaling constant typically has a range of 0.4 < 𝑒 < 0.9, for its valve, but 0.65 is believed to be the average value.

3.3.3.7 OPEX

It was explained that CAPEX may encompass a variety of different things. In this particular endeavor, the scope of OPEX is restricted to include just steam, power, cooling water utilities, and raw material. The needed solvents or mixtures of such solvents serve as the work's raw material.

In order to accomplish the goal of the reboiler, it is necessary for this project to provide steam utility. In addition, the cooling water utility meets the criteria for the condenser and lean amine cooler components. In conclusion, electricity is necessary for the operation of lean and rich pumps, as well as fans and compressors.

It is necessary to first explain a number of pertinent factors before moving on to the OPEX calculations for the simulated operations.

- Plant lifetime

The amount of time available is a crucial factor in any endeavor. Therefore, it needs to be taken into consideration while doing an economic analysis of the project. It is safe to presume that CO2 capture plants have a lifespan of twenty years based on the findings of several studies. Different recommendations have been made, which may be found in a few other books. Several studies, such as (Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation) and assumed a plant lifetime of 25 years.

For the sake of this study, the lifespan of the project is considered to be twenty years.

- Discount rate

The value of money changes throughout time. Additionally, carbon dioxide removal facilities are being built for a sizable time period. Therefore, it is appropriate to reflect changes in the worth of money during the course of the project.

Interest rates vary from 7% to 14%. In this study, the parameter is 7.5%, which is considered to remain constant during the project's duration (Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation). Other works have additional assumptions. For instance, (Kallevik, 2010) work assumes a 7% annual discount rate.

- Maintenance cost

Any expenses made by a person or organization to maintain the assets in excellent operating order are referred to as maintenance expenditures. An example of this kind of expense is paying money to maintain equipment.

Typically, it is expected that this cost will always represent a fixed portion of the overall installed cost (total CAPEX). This element fluctuates between 2% and 5%. 4% is recommended based on (Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation). (Hasan, Baliban, Elia, & Floudas, 2012) assumed this component as 5% of total CAPEX. To determine the OPEX cost for removal procedures, this part uses an assumption of 4%.

- Utilities cost

As was previously indicated, part of the machinery in the CO2 removal facilities needs utilities. Electricity is used for pumps and compressors, cooling water is used for condensers and lean amine coolers, and steam, which is the most significant utility, is used in the reboiler. Additionally, it was emphasized that utilities are included in the OPEX calculation because they are continually consumed.

Prices for the utilities listed vary across research. According to (Kallevik, 2010), the cost of power is equivalent to 0.4 (𝑁𝑂𝐾⁄𝑘𝑊ℎ), 25% of this amount for steam, 0.1 (𝑁𝑂𝐾⁄𝑘𝑊ℎ), and 0.033 (NOK/m3) for cooling water. Table 3.21 contains costs for the utilities used in this project.

Table 3.21: Price projections for the utility project

| Utility | Value | Unit |
| --- | --- | --- |
| Electricity | 0.132 | [𝑘𝑊ℎ] |
| Steam | 0.032 | [𝑘𝑊ℎ] |
| Cooling water | 0.022 | [𝑚3] |

### Annual hours of operation

The total number of hours that the whole facility is employed for all commercial reasons is referred to as the annual hours of operation.

Based on the capturing unit, this period of time differs among research. (Normann, Skagestad, Bierman, Wolf, & Mathisen, 2018) reported this parameter as 8322 [hours per year] for the steel sector, 7840 [hours per year] for pulp, 7320 [hours per year] for cement, and 8760 [hours per year] for silicon. Additionally, this value is expected to be 8000 [hours] each year based on steam production. This period was expected to be 7450 hours per year by (Khan, et al., 2020).

According to Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation), it is assumed that there are 8000 hours in a year (Orangi, Farsi Madan, Fajferek, Saeter, & Bahri, 2020).

- Cost of solvent

Three distinct solvents, MEA, MDEA, and PZ, have been employed in this investigation, as was already indicated. The two least priced amines are MEA and DEA. Below is a list of several amines' prices.

Table 3.22: Price of amine

| Amine | Value | Unit |
| --- | --- | --- |
| Diethylamine | 66.40 | [€/𝑙𝑖𝑡𝑟𝑒] |
| MEA | 30.50 | [€/𝑙𝑖𝑡𝑟𝑒] |
| PZ | 68.70 | [€/𝑙𝑖𝑡𝑟𝑒] |
| EDA | 31.70 | [€/𝑙𝑖𝑡𝑟𝑒] |
| MDEA | 51.60 | [€/𝑙𝑖𝑡𝑟𝑒] |
| DEA | 25.70 | [€/𝑙𝑖𝑡𝑟𝑒] |

In this work, the price for MEA is extracted from (Aromada, Eldrup, Normann, & Øi, 2020) work, 2069 [𝑚3].

Cost for MDEA and PZ is calculated proportionally from table 3.22 which lead to 4660 [𝑚3] for PZ and 3494 [𝑚3] for MDEA.

3.3.3.8 Estimating costs for Simulated instances

When it comes to CAPEX, each simulated instance is, first and foremost, stated in terms of size, material, and number-based specifications. Aspen In-Plant Cost Estimation version 10 was also used in order to ascertain the costs associated with the plant's carbon dioxide removal equipment.

Additionally, each procedure was modified in terms of the installation cost factor depending on the EDF technique and the materials that were provided. The overall CAPEX was calculated by adding together the costs of each individual piece of plant machinery and equipment.

As was indicated, the calculation of OPEX necessitates taking a number of different aspects into consideration. In section 5.7, a concise explanation was provided for those that were significant. The parameters, together with their explanations, are shown in table 3.23 below.

Table 3.23: Important factors to consider when calculating OPEX

| Parameter | Value | Unit |
| --- | --- | --- |
| Plant lifetime | 20 | [year] |
| Discount rate | 7.5% | - |
| Maintenance cost | 4% of total CAPEX | - |
| Electricity cost | 0.132 | [kWh] |
| Steam cost | 0.032 | [kWh] |
| Cooling water cost | 0.022 | [m3] |
| MEA cost | 2069 | [m3] |
| MDEA cost | 3494 | [m3] |
| PZ cost | 4660 | [m3] |
| Annual operational time | 8000 | [hours/year] |
| CEPCI in 2016 | 542 | - |
| CEPCI in 2020 | 655.7 | - |
| Scaling constant | 0.65 | - |

Because OPEX pertains to day-to-day expenditures, it has to be computed for the whole year (Orangi , Farsi Madan, Fajferek, Saeter, & Bahri, 2020). Equation 3.25 is used in order to arrive at an estimate for each variable operating cost (Aromada, Eldrup, Normann, & Øi 2020).

𝑉𝑎𝑟𝑖𝑎𝑏𝑙𝑒 𝑜𝑝𝑒𝑟𝑎𝑡𝑖𝑛𝑔 𝑐𝑜𝑠𝑡 (𝑦𝑒𝑎𝑟) = 𝑐𝑜𝑛𝑠𝑢𝑚𝑝𝑡𝑖𝑜𝑛 (unit⁄ℎ𝑜𝑢𝑟) × 𝑢𝑛𝑖𝑡 𝑝𝑟𝑖𝑐𝑒 (𝑢𝑛𝑖𝑡) × 𝑜𝑝𝑒𝑎𝑡𝑖𝑜𝑛𝑎𝑙 ℎ𝑜𝑢𝑟𝑠 (ℎ𝑜𝑢𝑟⁄𝑦𝑒𝑎𝑟)            (3.25)

The total CAPEX for the lifespan of each process has been determined. Therefore, you need to do the calculation for each year. This may be accomplished by the use of the annualised factor (Aromada, Eldrup, Normann, & Øi 2020).

Annualized factor=1n1(1+i)n                        (3.26)

where, 𝑖 indicates the interest rate and 𝑛 is plant lifetime.

For the sake of this study, we will assume that the interest rate will remain the same throughout all of the computation years: 7.5%. In addition, the lifespan of a plant is estimated to be twenty years.

It is important to note that in certain studies, only a negligible portion of the plant's lifespan is allocated to the building period. For example, in (Aromada, Eldrup, Normann, & Øi 2020), the overall lifespan of the plant was expected to be 25 years, with the first two years being devoted to its development. This work does not incorporate the aforementioned assumption.

The application of the annualized factor to the total CAPEX ultimately results in the annualized CAPEX.

Annualized CAPEX=Total CAPEXAnnualized factor          (3.27)

3.3.3.9 Economy Analysis

The total yearly cost is used to determine the economy of each simulated process, taking into account:

𝑡𝑜𝑡𝑎𝑙 𝑎𝑛𝑛𝑢𝑎𝑙 𝑐𝑜𝑠𝑡 (Currency/year) = 𝑎𝑛𝑛𝑢𝑎𝑙𝑖𝑧𝑒𝑑 𝐶𝐴𝑃𝐸𝑋 (𝑦𝑒𝑎𝑟) + 𝑎𝑛𝑛𝑢𝑎𝑙 𝑂𝑃𝐸𝑋 (𝑦𝑒𝑎𝑟)                (3.28)

In the present study, the basic case is taken to be a reference so that the total yearly cost of additional processes that require MDEA and PZ or their mixes may be compared. These other processes include the usage of solvents.

The cost of CO2 capture is another factor that may be considered when contrasting various procedures from an economic point of view. This cost is described as:

Annual cost of captured Co2=total annual cost (Rs/year)mass flow rate of captured Co2(kg/year)                 (3.29)

Both of the established metrics in 3.28 and 3.29 are utilized to determine whether or not there has been an economic improvement as a result of using solvents or their blends as opposed to the base scenario, which utilizes MEA at a concentration of 30 weight percent.

As was indicated previously, the method of removing CO2 using amines is an energy-intensive process that utilizes a significant amount of energy in the reboiler for the purpose of regeneration. Therefore, one may get this parameter by the use of a simulated process.

regeneration energy=applied steam in proces MJhMass flow rate of caprture CO2kgh      MJkg         (3.30)

This parameter is also computed for all simulated processes in order to determine the impact of additional solvents outside the basic case and the blends of those solvents on this term.

3.3.3.10 Guidelines for Aspen In-Plant Cost Estimator cost estimating

The work that is being done right now to analyze the economy has been met with certain challenges. These issues, along with some potential solutions to them, will be enumerated and discussed in this section.

- Because of the high value of the flue gas, the diameter of the absorber that was predicted to be necessary was rather large—around 20 metres. Having an absorber of such a massive scale in order to achieve the desired outcome of the operation comes at a significant financial expense to the facility. Therefore, it is not unreasonable to have smaller things that take up the same amount of space in the packaging. The large absorber is cut up into sixteen smaller absorbers as part of this procedure.

- Because it is one of the most significant pieces of machinery in the plant, the lean rich heat exchanger has to have a big contact area in order to fulfil the objectives of the plant. It is impossible to heat this large of an area with just one heat exchanger, thus it is best to look into using many, smaller ones. The relevant assumption is one 1000 m2 per shell. Therefore, the huge area of the lean rich heat exchanger is split into many smaller areas, each of which is the same as the others.

- As was remarked upon before, the flue gas flow rate in this operation is rather high. Therefore, the supposed fan for the process has to be strong enough, and as a result, its size needs to be increased, in order to provide the appropriate level of driving force. Due to the size of the fan required for this task, an "out of range size" mistake was made in the cost calculation. This mistake may be remedied by supposing the operation of two parallel fans that are both smaller and similar in design.

- In most of the studies, just one of the examples is looked at from an economic perspective, while the rest of the cases are computed using the power law. This procedure could not be much easier, but the findings would be subject to a far larger degree of uncertainty. Inside the scope of this job, comprehensive estimates are provided for each and every simulated situation, as well as for every piece of applicable equipment within the plant. A method of this kind requires more time, but the results are more accurate.

- The major pieces of plant equipment are the only ones that count toward the economy of the current job. There are a few more things that might be considered for CAPEX that are more realistic. In addition, regarding the OPEX, it is necessary to include human resources such as employees and engineers in the calculations.

REFERENCES

- AACE International, Skills & Knowledge of Cost Engineering. 2007: Morgantown, WV 26501 USA.

- Abu-Zahra, M. R., Schneiders, L. H., Niederer, J. P., Feron, P. H., & Versteeg, G. F. (2007). A parametric study of the technical performance baced on monoethanolamine. GREENHOUSE GAS CONTROL, 37-46.

- Ali, H., Eldrup, N. H., Normann, F., Skagestad, R., & Øi, L. E. (2019). Cost Estimation of CO2 Absorbtion Plants for CO2 Mitigation - Method and Assumptions. Greenhouse Gas Control, 88, 10-23.

- Ali, H., et al., Cost estimation of heat recovery networks for utilization of industrial excess heat for carbon dioxide absorption. International Journal of Greenhouse Gas Control, 2018. 74: p. 219-228.

- Arachchige, U. S., & Melaaen, M. C. (2012). Blended Amines' Effect on Post Combustion CO2 Capture. Environment Poluution and Remediation. Montreal, Quebec, Canada: ResearchGate.

- Arachchige, U. S., & Melaaen, M. C. (2012). Selection of Packing Material for Gas Absorption. European Journal of Scientific Research, 87, 117-126. Retrieved from http://www.europeanjournalofscientificresearch.com/

- Aromada, S. A., Eldrup, N. H., Normann, F., & Øi, L. E. (2020). Simulation and Cost Optimization of different Heat Exchnagers for CO2 Capture. SIMS2020.

- Brickett, L. (2015). CARBON DIOXIDE CAPTURE HANDBOOK. National Energy Technology Laboratory. Retrieved from www.netl.doe.gov

- Cavallo , C. (2011). thomasnet.com. (Thomas Industry) Retrieved from https://www.thomasnet.com/articles/process-equipment/shell-and-tube-heat-exchangers/

- Gerrard, A.M., Guide to Capital Cost Estimating. 2000, Warwickshire, UK: Institute of Chemical Engineers (IChemE).

- Jilvero, H., et al., Techno-economic Analysis of Carbon Capture at an Aluminum Production Plant – Comparison of Post-combustion Capture Using MEA and Ammonia. Energy Procedia, 2014. 63: p. 6590-6601

- Jilvero, H., et al., Techno-economic evaluation of an ammonia-based postcombustion process integrated with a state-of-the-art coal-fired power plant. International Journal of Greenhouse Gas Control, 2014. 31: p. 87-95.

- Kallevik, O. B. (2010). Cost estimation of CO2 removal in HYSYS. Porsgrunn: Universirty of SouthEastern Norwegian

- Mathisen, A., et al., Cost Optimised CO2 Capture from Aluminium Production. Energy Procedia, 2014. 51: p. 184-190.

- Moshfeghian, M. (2015, September 15). PetroSkills. Retrieved from http://www.jmcampbell.com/tip-of-the-month/2015/09/gas-liquid-separators-sizingparameter/

- Øi, L.E., et al., Simulation and Cost Comparison of CO2 Liquefaction. Energy Procedia, 2016. 86: p. 500-510.

- Øi, L.E., Removal of CO2 from exhaust gas, in Department of Process, Energy and Environmental Technology. 2012, Telemark University College.

- Orangi , S., Farsi Madan, F., Fajferek, K. G., Sæter, N. T., & Bahri, S. (2020). Process simulation and cost estimation of CO2 capture in Aspen HYSYS using different estimation methods. Project cource, University of South-Eastern Norwegian, Porsgrunn.

- Park, K. and L.E. Øi, Optimization of gas velocity and pressure drop in CO2 absorption column. Linköping Electronic Conference Proceedings, 2017. SIMS58: p. 292-297.

- Seader, J. D., & Henley, E. J. (2006). Absorption and Stripping . In Separation Process Principles (p. 194). Wiley.

- Smith, R., Chemical Process Design and Integration. 2005, England: John Wiley & Sons Ltd.

- Wong, S., Module 4: CO2 compression and transportation to storage sites, in in Building capacity for CO2 Capture and Storage in the APEC region: A training manual for policy makers and practitioners. 2012, APEC: Singapore.

- Yu, F., Process design for chemical engineers. 2014: Ten Books, Inc.