SEHS3305 Linear systems and control

Experiment

Title

Computer-aided symbolic computation and digital simulation of control systems

Object

Basic skills of MATLAB

Symbolic calculation

Simulink manipulation

Brief introduction

What is MATLAB?

MATLAB is a computer program that combines computation and visualization power that makes it particularly useful tool for engineers.

MATLAB is an executive program, and a script can be made with a list of MATLAB commands like other programming language.

 MATLAB Stands for MATrix LABoratory.

The system was designed to make matrix computation particularly easy.

 The MATLAB environment allows the user to:

manage variables

import and export data

perform calculations

generate plots

develop and manage files for use with MATLAB.

Type one of following commands in the command window to get help : help – lists all the help topic help topic – provides help for the specified topic help command – provides help for the specified command help help – provides information on use of the help command helpwin – opens a separate help window for navigation lookfor keyword – Search all M-files for keyword

Problems with MATLAB command window

Please solve the following problems with MATLAB, each of the problem should have relating codes with comment in a two-column format. After the code, a capture of MATLAB window should present the result.

Part A Polynomials

Consider the two polynomials  and . Compute

(1)

(2) Roots of  and

(3) and

Part B Partial fraction

Find the partial fraction of the following expressions.

(1)

(2)

Part C Laplace transform

(1) Find the Laplace transform of the following expressions.

(a)

(b)

(2) Find the Inverse Laplace transform of the following expressions.

(a)

(b)

Problems in Simulink toolbox

(1) Consider the control system shown in the figure, Draw unit-step response curves for k=5,10 and 20. Discuss the results.

(2) Consider the tachometer feedback control system shown in the figure, Obtain the unit step response curves when the feedback variable k assumes five values:  Discuss the results.

(Due date of lab report submission 21 Apr)

SIMULINK

Simulink provides access to an extensive set of blocks that accomplish a wide range of functions useful for the simulation and analysis of dynamic systems. The blocks are grouped into libraries, by general classes of functions.

- Mathematical functions such as summers and gains are in the Math library.

- Integrators are in the Continuous library.

- Constants, common input functions, and clock can all be found in the Sources library.

- Scope, To Workspace blocks can be found in the Sinks library.

Simulink is a graphical interface that allows the user to create programs that are actually run in MATLAB. When these programs run, they create arrays of the variables defined in Simulink that can be made available to MATLAB for analysis and/or plotting. The variables to be used in MATLAB must be identified by Simulink using a “To Workspace” block, which is found in the Sinks library. (When using this block, open its dialog box and specify that the save format should be Matrix, rather than the default, which is called Structure.) The Sinks library also contains a Scope, which allows variables to be displayed as the simulated system responds to an input. This is most useful when studying responses to repetitive inputs.

Simulink uses blocks to write a program. Blocks are arranged in various libraries according to their functions. Properties of the blocks and the values can be changed in the associated dialog boxes. Some of the blocks are given below.

SUM (Math library)

A dialog box obtained by double-clicking on the SUM block performs the configuration of the block, allowing any number of inputs and the sign of each. The sum block can be represented in two ways in Simulink, by a circle or by a rectangle. Both choices are shown

Figure 1: Two Simulink blocks for a summer representing y = x 1+ x2 – x3

GAIN (Math library)

A gain block is shown by a triangular symbol, with the gain expression written inside if it will fit. If not, the symbol - k - is used. The value used in each gain block is established in a dialog box that appears if the user double-clicks on its block.

Figure 2: Simulink block for a gain of K.

INTEGRATOR (Continuous library)

The block for an integrator as shown below looks unusual. The quantity 1/s comes from the Laplace transform expression for integration. When double-clicked on the symbol for an integrator, a dialog box appears allowing the initial condition for that integrator to be specified. It may be implicit, and not shown on the block, as in Figure (a). Alternatively, a second input to the block can be displayed to supply the initial condition explicitly, as in part

- of Figure 3. Initial conditions may be specific numerical values, literal variables, or algebraic expressions.

Figure3: Two forms of the Simulink block for an integrator.

  - Implicit initial condition. (b) Explicit initial condition.

CONSTANTS (Source library)

Constants are created by the Constant block, which closely resembles Figure 4. Double- clicking on the symbol opens a dialog box to establish the constant’s value. It can be a number or an algebraic expression using constants whose values are defined in the workspace and are therefore known to MATLAB.

Figure 4: A constant block

STEP (Source library)

A Simulink block is provided for a Step input, a signal that changes (usually from zero) to a specified new, constant level at a specified time. These levels and time can be specified through the dialog box, obtained by double-clicking on the Step block.

Figure 5: A step block

SIGNAL GENERATOR (Source library)

One source of repetitive signals in Simulink is called the Signal Generator. Double-clicking on the Signal Generator block opens a dialog box, where a sine wave, a square wave, a ramp (sawtooth), or a random waveform can be chosen. In addition, the amplitude and frequency of the signal may be specified. The signals produced have a mean value of zero. The repetition frequency can be given in Hertz (Hz), which is the same as cycles per second, or in radians/second.

Figure 6: A signal generator block

SCOPE (Sinks library)

The system response can be examined graphically, as the simulation runs, using the Scope block in the sinks library. This name is derived from the electronic instrument, oscilloscope, which performs a similar function with electronic signals. Any of the variables in a Simulink diagram can be connected to the Scope block, and when the simulation is started, that variable is displayed. It is possible to include several Scope blocks. Also it is possible to display several signals in the same scope block using a MTJX block in the signals & systems library. The Scope normally chooses its scales automatically to best display the data.

Figure 7: A scope block with MUX block

Two additional blocks will be needed if we wish to use MATLAB to plot the responses versus time. These are the Clock and the To Workspace blocks.

CLOCK (Sources library)

The clock produces the variable “time” that is associated with the integrators as MATLAB calculates a numerical (digital) solution to a model of a continuous system. The result is a string of sample values of each of the output variables. These samples are not necessarily at uniform time increments, so it is necessary to have the variable “time” that contains the time corresponding to each sample point. Then MATLAB can make plots versus “time.” The clock output could be given any arbitrary name; we use “t” in most of the cases.

Figure 8: A clock block

To Workspace (Sinks library)

The To Workspace block is used to return the results of a simulation to the MATLAB workspace, where they can be analyzed and/or plotted. Any variable in a Simulink diagram can be connected to a ToWorkspace block. In our exercises, all of the state variables and the input variables are usually returned to the workspace. In addition, the result of any output equation that may be simulated would usually be sent to the workspace. In the block parameters drop down window, change the save format to ‘array’.

Figure 9: A To Workspace block

In the Simulink diagram, the appearance of a block can be changed by changing the foreground or background colours, or by drop shadow or other options available in the format drop down menu. The available options can be reached in the Simulink window by highlighting the block, then clicking the right mouse button. The Show Drop Shadow option is on the format drop-down menu.

Simulink provides scores of other blocks with different functions.

#### You are encouraged to browse the Simulink libraries and consult the online Help facility provided with MATLAB.